HL7_DAM_ANESTH_R1_I1_2020FEB

HL7 Domain Analysis Model:

Intra-Procedure Anesthesia,

Release 1

February 2020

HL7 Informative Ballot

Sponsored by:
Anesthesia Work Group

Healthcare Devices Work Group

Anesthesia Co-Chairs

Martin Hurrell PhD

John Walsh, MD

Copyright © 2019-2020 Health Level Seven International ® ALL RIGHTS RESERVED. The reproduction of this material in any form is strictly forbidden without the written permission of the publisher.  HL7 and Health Level Seven are registered trademarks of Health Level Seven International. Reg. U.S. Pat & TM Off.

Use of this material is governed by HL7's IP Compliance Policy.

IMPORTANT NOTES:

HL7 licenses its standards and select IP free of charge. If you did not acquire a free license from HL7 for this document, you are not authorized to access or make any use of it. To obtain a free license, please visit http://www.HL7.org/implement/standards/index.cfm.

If you are the individual that obtained the license for this HL7 Standard, specification or other freely licensed work (in each and every instance "Specified Material"), the following describes the permitted uses of the Material.

A. HL7 INDIVIDUAL, STUDENT AND HEALTH PROFESSIONAL MEMBERS, who register and agree to the terms of HL7’s license, are authorized, without additional charge, to read, and to use Specified Material to develop and sell products and services that implement, but do not directly incorporate, the Specified Material in whole or in part without paying license fees to HL7.

INDIVIDUAL, STUDENT AND HEALTH PROFESSIONAL MEMBERS wishing to incorporate additional items of Special Material in whole or part, into products and services, or to enjoy additional authorizations granted to HL7 ORGANIZATIONAL MEMBERS as noted below, must become ORGANIZATIONAL MEMBERS of HL7.

B. HL7 ORGANIZATION MEMBERS, who register and agree to the terms of HL7's License, are authorized, without additional charge, on a perpetual (except as provided for in the full license terms governing the Material), non-exclusive and worldwide basis, the right to (a) download, copy (for internal purposes only) and share this Material with your employees and consultants for study purposes, and (b) utilize the Material for the purpose of developing, making, having made, using, marketing, importing, offering to sell or license, and selling or licensing, and to otherwise distribute, Compliant Products, in all cases subject to the conditions set forth in this Agreement and any relevant patent and other intellectual property rights of third parties (which may include members of HL7). No other license, sublicense, or other rights of any kind are granted under this Agreement.

C. NON-MEMBERS, who register and agree to the terms of HL7’s IP policy for Specified Material, are authorized, without additional charge, to read and use the Specified Material for evaluating whether to implement, or in implementing, the Specified Material, and to use Specified Material to develop and sell products and services that implement, but do not directly incorporate, the Specified Material in whole or in part.

NON-MEMBERS wishing to incorporate additional items of Specified Material in whole or part, into products and services, or to enjoy the additional authorizations granted to HL7 ORGANIZATIONAL MEMBERS, as noted above, must become ORGANIZATIONAL MEMBERS of HL7.

Please see http://www.HL7.org/legal/ippolicy.cfm for the full license terms governing the Material.

Ownership. Licensee agrees and acknowledges that HL7 owns all right, title, and interest, in and to the Materials. Licensee shall take no action contrary to, or inconsistent with, the foregoing.

Licensee agrees and acknowledges that HL7 may not own all right, title, and interest, in and to the Materials and that the Materials may contain and/or reference intellectual property owned by third parties (“Third Party IP”).  Acceptance of these License Terms does not grant Licensee any rights with respect to Third Party IP. Licensee alone is responsible for identifying and obtaining any necessary licenses or authorizations to utilize Third Party IP in connection with the Materials or otherwise. Any actions, claims or suits brought by a third party resulting from a breach of any Third-Party IP right by the Licensee remains the Licensee’s liability.

Following is a non-exhaustive list of third-party terminologies that may require a separate license:

| Terminology | Owner/Contact |
| --- | --- |
| Current Procedures Terminology (CPT) code set | American Medical Association https://www.ama-assn.org/practice-management/cpt-licensing |
| SNOMED CT | SNOMED International http://www.snomed.org/snomed-ct/get-snomed-ct or info@ihtsdo.org |
| Logical Observation Identifiers Names & Codes (LOINC) | Regenstrief Institute |
| International Classification of Diseases (ICD) codes | World Health Organization (WHO) |
| NUCC Health Care Provider Taxonomy code set | American Medical Association. Please see www.nucc.org. AMA licensing contact: 312-464-5022 (AMA IP services) |

| Authors: | Martin Hurrell, Andrew Norton, Ellen Torres, John Walsh |
| --- | --- |
| Domain Experts: | Steven Dain, Grant Forrest, Norman Jones, Andrew Marchant, Terri Monk, Andrew Norton, Ellen Torres, John Walsh |
| Thanks to: | John Rhoads, Koichiro Matsumoto, Patrick.Mccormick, Brian Rothman, Vikas O'Reilly-Shah, Ioana Singureanu |

Contents

Introduction	1

1	Scope and intent	1

HL7 and other standards	1

Terminology	2

Future release	2

Behavioral Component	3

1	Actors	3

Actors (Human)	3

Actors (Non-human)	4

2	Preparation for procedure	6

3	Intra-procedural overview	11

Major procedure example: Repair of aortic aneurism	17

4	Generic Clinical Procedures	22

Positioning	23

Aseptic Technique	26

Site Preparation	26

Local Anesthetic Infiltration	27

Field Block	28

Device Guidance & Confirmation of Position	29

Anatomical Landmarks	29

Loss of resistance technique	29

Imaging Guidance	29

Nerve Stimulation	30

Seldinger Technique	30

Observation of Sampled Substance	34

Test Dose	34

Invasive Device Placement	34

Tunneled placement	35

5	Mainstream Procedures	37

Induction of general anesthesia	37

Monitoring	39

Train of Four (ToF) monitoring	41

6	Vascular Access	44

Central Venous Catheter (CVC)	45

Peripheral Venous Catheter (PVC)	47

Arterial Line (A-line)	49

7	Airway Management	51

Routine laryngoscopic endotracheal intubation	51

8	Regional Anesthesia	54

Epidural	54

9	Respiratory Support	59

Pre-oxygenation	59

Information component	62

Modes of Ventilation	62

Outline of respiratory support system	74

Gases (Class)	74

AnesthesiaWorkstation (Class)	74

VolatileAgentDeliveryDevice (Class)	75

Monitor (Class)	75

Ventilator (Class)	75

Breathing System (Class)	76

Airway (Class)	76

Patient ConnectionPort (Class)	76

BagAndMask (Class)	77

Procedure	79

Practitioner and Care Team	84

Checks & Safety	90

Observation	101

Findings and Events	108

AllergyIntolerance	116

Site Preparation	118

Positioning	121

Aseptic Technique	126

Kit	127

Device	128

Needle	134

Catheter	137

Tube	139

Mask	141

Laryngoscope	144

Lumen	145

Positioning Aid	148

Device Settings	149

Device Use	152

DeviceUse (Class)	153

Medication	156

Medication Administration	159

Vocabulary Examples	165

Administrative procedure	165

Patient Transfer	167

Airway device	168

Intravenous anesthetic agent	169

Local anesthetic nerve block in lower limb (enumeration)	170

# Introduction

## Scope and intent

The anesthesia care process can be roughly divided into three phases:

- Anesthesia Pre-assessment

- Intra-procedural anesthesia

- Post-anesthesia care

Anesthesia Pre-assessment was the topic of a previous DAM, ‘HL7 Version 3 Domain Analysis Model: Preoperative Anesthesiology, Release 1’.

The current DAM does not duplicate this material. A relatively small subset of findings from the pre-assessment have relevance to the management of the current anesthetic procedure. Anesthesia Plans are also typically developed with reference to the pre-assessment.

The document aims to provide a reasonable coverage of intra-procedural anesthesia -  its content is intended to support the conventional anesthesia record. In the course of its development the authors have reviewed a large number of anesthesia records, mainly from the US and the UK in order to identify requirements.

Many of the model elements in this DAM are already well, and more fully, defined by other HL7 work groups and standards. An example, in relation to monitoring is ‘Detailed Clinical Model for Medical Devices’ (V3DAM_DCM4MEDDEV_R1_I1_2011MAY) produced by the Healthcare Devices Workgroup. Devices are also exhaustively dealt with in ISO/IEEE 11073.

This has lead us to limit both the number of procedural examples and coverage of some topics. For example physiological / vital signs monitoring is not dealt with in great detail despite its importance in intra-procedural anesthesia because we have felt that this is better done by other work groups who have more expertise and have already produced relevant material e.g. Health Care Devices and Patient Care.

On the subject of devices, it is increasingly the case that anesthesia information management systems support methods that allow the identity and other detailed characteristics of a device, such as its operational settings, to be recorded with little effort. Further, by linking this to device definitions even more information may easily be ascertained. It follows that in a modern IT environment the electronic record may incorporate, or reference, much more information than is captured in paper records and so a model based on the content of current manual anesthesia records is likely to fall short.

We have based the modelling of some key generic elements such as devices, procedures, observations and medication administrations on the relevant FHIR resources because they cover the requirements extremely well.

### Terminology

In 2003 the Anesthetic Patient Safety Foundation (APSF) established a Data Dictionary Task Force (DDTF) which later became known as the International Organization for Terminology in Anesthesia (IOTA). The aim was to create a vocabulary for the domain of anesthesia. The current term set contains just under 5,000 concepts. A comparatively few of these are included at the end of this release, as examples. The terms are now maintained by the SNOMED CT Anesthesia Clinical Reference Group (CRG).

SNOMED CT is free to use for healthcare providers in Member states.  Developers implementing SNOMED CT in clinical applications require an affiliate license which is provided free of charge from SNOMED International where the intended application is for use in Member States. Usage of SNOMED CT should be registered through the relevant National Release Centre (NRC) unless the country has access to the SNOMED Member Licensing and Distribution Service (MLDS) when this service should be used instead.

Annual licenses will be required to use SNOMED CT in non-Member states which can be obtained through MLDS . The scale of charges is based on World Bank Country Income bands.

There are a number of situations where licenses are granted free of charge including World Bank Country Income Category E (low income) economies and for various charitable and research purposes.

Further information on SNOMED CT licensing can be found at https://www.snomed.org/snomed-ct/get-snomed.

The anesthesia term set was authored and is maintained using Protégé-OWL. The current version can be found at: https://confluence.ihtsdotools.org/pages/viewpage.action?pageId=38260021

### Future release

In a future release it is hoped to: increase the number of use cases, particularly for airway procedures and regional anesthesia, create detailed term bindings using the IOTA term set, and define elements of the anesthesia record (clinical notes) with links to the model.

# Behavioral Component

## Actors

### Actors (Human)

This package contains identities and definitions of human actors referred to in the model

| Actor | Description |
| --- | --- |
| Resident | In many teaching hospitals, resident doctors in training and medical students may be a part of the surgical team. |
| Surgeon | A surgeon has finished 4 years of medical school and 4 or more years of special training after medical school. Most surgeons have passed exams for board certification. The American Board of Surgery is the national group that gives this certification for general surgery in the U.S. |
| Physician Assistant | Physician assistants practice medicine under the supervision of a doctor. In the UK Physician Assistants are called Surgical Care Practitioners (who often have a nursing background but may be technically trained). Physician Assistants may act as an assistant to the surgeon. Or they may, for example, close incisions with stitches (sutures) or staples. |
| Procedural Assistant | An appropriately qualified person who assists another to perform an act |
| Surgeon - Resident | A surgical resident has finished medical school and is now doing further specialty training in surgery |
| Surgical Tech | Surgical techs assist with the surgery by setting up a sterile operating room. They get supplies and surgery tools ready. And they hand the surgeon the tools he or she asks for. They must pass an exam to be certified by the National Board of Surgical Assisting (NBSTA). |
| Surgeon - Attending | A surgical member of the attending staff of a hospital |
| Surgeon - Fellow | A surgeon who has completed their residency and elects to complete further training |
| Anesthesiologist | Anesthesiologists are physicians specializing in perioperative care, developing anesthetic plans, and the administration of anesthetics. The anesthesiologist takes part in all 3 phases of surgery: before, during, and after. |
| CRNA | Certified Registered Nurse Anesthetists (CRNAs) are advanced practice registered nurses with graduate level education who have a high degree of autonomy. CRNAs provide anesthetics to patients in every practice setting, and for every type of surgery or procedure. They are often the sole anesthesia providers in rural hospitals.. |
| ACSP | Authorized Clinical Service Provider: a clinician of any title who is permitted to perform the focus procedure. This role may be played by an anesthesiologist, physician, nurse anesthetist (CRNA), nurse or other qualified person. Permission may be assigned at a federal, state or local level This role is assigned to the care team member who performs the procedure. |
| Anesthesia Care Team | This may be any practitioner who is specialized in providing anesthesia care e.g. Anesthesiologist, CRNA, Anesthesia Resident, Anesthesia Nurse or, for example, in the UK, an Operating Department Practitioner (ODP) |
| Patient | A person who is the recipient of healthcare services. In this use case, a Patient is someone scheduled for a surgery or procedure that requires the provision of anesthesia services, who is undergoing pre-anesthesia evaluation. |
| Perfusionist | A perfusionist, also known as a Clinical Perfusionist, Cardiovascular Perfusionist, Clinical Perfusion Scientist, or Medical Perfusionist, is a healthcare professional who uses the cardiopulmonary bypass machine (heart–lung machine) during cardiac surgery and other surgeries that require cardiopulmonary bypass to manage the patient's physiological status. |
| Nurse | A credentialed registered nurse (RN), licensed practical nurse (LPN), or nursing assistant (NA) who works in the clinic and who may perform some of the tasks involved in pre-anesthesia evaluation. An RN has a broader scope of practice than an LPN, who has a broader scope of practice than an NA. Although most tasks could be performed by the Screening Clinician, some settings may utilize a Nurse for tasks that do not require diagnosis or medical decision-making, such as obtaining vitals and performing pre-operative teaching. |
| Operating Room Nurse | The operating room nurse is a registered nurse who assists the surgeon during surgery. Operating room nurses are certified in various areas of surgery. |

### Actors (Non-human)

| Actor | Description |
| --- | --- |
| EHR | An electronic health record (EHR), or electronic medical record (EMR), is the systematized collection of patient and population electronically-stored health information in a digital format. These records can be shared across different health care settings. Records are shared through network-connected, enterprise-wide information systems or other information networks and exchanges. EHRs may include a range of data, including demographics, medical history, medication and allergies, immunization status, laboratory test results, radiology images, vital signs, personal statistics like age and weight, and billing information. |
| Patient Monitor | Patient monitors measure, record, distribute and display combinations of biometric values such as heart rate, SPO2, blood pressure, temperature and more. High-capability, multi-function monitors are typically used in hospitals and clinics to ensure a high-level of quality patient care. |
| AIMS | An Anesthetic Information Management System (AIMS) is a computer-based system that gathers , displays, stores and prints information relevant to the anesthetic procedure including all information that would normally be found in the manual anesthetic record. AIMS differ in capabilities but most gather data directly from patient-connected devices such as patient monitors, anesthetic ventilators and syringe pumps. Other data are generally entered by the anesthesiologist. AIMS may also communicate with other IT systems such as the EHR or Clinical Laboratory. Data include: Information about the case, patient, care teams, type of surgical and anesthetic procedures planned and done etc. Detailed information about equipment, checklists and specific interventions such as the insertion of an IV cannula, induction, intubation, regional anesthesia etc. - in short, any information relevant to the anesthetic procedure. Administration of drugs and fluids Data from patient connected devices: anesthetic machine, ventilator, syringe pumps, physiological monitors Observations Events including milestone events and adverse events |
| Anesthetic Machine / Ventilator | Over the years, the conventional anaesthesia machine has evolved into an advanced carestation. The new machines use advanced electronics, software and technology to offer extensive capabilities for ventilation, monitoring, inhaled agent delivery, low-flow anaesthesia and closed-loop anaesthesia. They offer integrated monitoring and recording facilities and seamless integration with anaesthesia information systems. It is possible to deliver tidal volumes accurately and eliminate several hazards associated with the low pressure system and oxygen flush. Appropriate use can result in enhanced safety and ergonomy of anaesthetic delivery and monitoring. Divatia, J., Patil, V. and Shetmahajan, M., 2013. The modern integrated anaesthesia workstation. Indian Journal of Anaesthesia, 57(5), pp.446 - 454 |

## Preparation for procedure

Figure 1 : Preparation

| Actor | Description |
| --- | --- |
| Patient | A person who is the recipient of healthcare services. In this use case, a Patient is someone scheduled for a surgery or procedure that requires the provision of anesthesia services, who is undergoing pre-anesthesia evaluation. |
| Anesthesia Care Team | This may be any practitioner who is specialized in providing anesthesia care e.g. Anesthesiologist, CRNA, Anesthesia Resident, Anesthesia Nurse |
| Anesthesiologist | Anesthesiologists are physicians specializing in perioperative care, developing anesthetic plans, and the administration of anesthetics. The anesthesiologist takes part in all 3 phases of surgery: before, during, and after. |

Transfer patient

In some countries, it is customary to prepare the patient in a location close to, or adjoining the OR in which case it is necessary to transfer them to the OR for the procedure

Edit plan

Typically, a plan for the anesthetic management of the case will have been formulated as part of the pre-operative assessment. In this case the plan requires simply to be reviewed. However, changes may require to be made in the light of recently added information or a new plan may require to be created on the basis of the review of the pre-operative assessment.

Pre-Assessment

The purpose of the pre-assessment is:

- To document the diagnoses/ condition(s) for which surgery is needed.

- Assess the patient’s overall health status.

- Identify conditions that could cause problems both during and after surgery e.g. allergies, sleep apnea, airway issues, previous untoward events

- Assess perioperative risks

- Optimize the patient’s medical condition to reduce surgical and anesthetic peri-operative morbidity or mortality.

- Develop an appropriate perioperative care plan.

- Discuss with the patient the proposed surgery, anesthesia, intraoperative care and postoperative pain treatments and record consent.

A detailed discussion of anesthetic pre-assessment is out-of-scope of this DAM but is given in 'HL7 Version 3 Domain Analysis Model: Preoperative Anesthesiology, Release 1’.

The anesthetic pre-assessment may be carried out several weeks before the planned procedure, but in the case of urgent and emergency surgery may only be carried out hours or minutes before. Also, new information may be added to the pre-assessment at any time, if, for example, there has been a recent change in current medications, laboratory data or health status.

The pre-assessment includes a risk assessment and anesthetic plan. It also includes a record of consent given by the patient or their representative following a discussion of the planned procedure(s) and their associated risks.

This is an essential precursor to obtaining consent because the patient has to understand the benefits and hazards of the proposed procedure in order for consent to be meaningful. It may include a discussion of a procedure that is possible, but not planned. For example, it might be planned to use mild sedation and an epidural block. However, in the case that the block could not be established then the anesthesiologist might need to induce general anesthesia so this would be discussed as part of the plan.

The Review of the pre-assessment is to refresh the awareness of the anesthesiologist and other members of the care team, of issues that have a bearing on the management of the case.

This information flows forward to the intra-procedural phase.

Anesthetic Record

Throughout the procedure a comprehensive record is maintained. Today, this is likely to be an electronic record that is made using an Anesthesia Information Management System (AIMS). It includes:

- Information about the case, patient, care teams, type of surgical and anesthetic procedures planned and done etc.

- Detailed information about equipment, checklists and specific interventions such as the insertion of an IV cannula, induction, intubation, regional anesthesia etc. - in short, any information relevant to the anesthetic procedure.

- Administration of drugs and fluids

- Data from patient connected devices: anesthetic machine, ventilator, syringe pumps, physiological monitors

- Observations

- Events including milestone events and adverse events

Case Summary

This is the first element of the anesthetic record. It identifies

- the patient, their date of birth and gender and potentially other information such as address and details of next of kin

- the Practitioners in the Care Team (Anesthetic and Surgical) including roles and supervision

- the planned surgical procedure(s) and associated pre-procedure diagnoses. At the end of the case, any procedure(s) not done, together with any unplanned procedure(s) that were, are recorded with a note of the reasons why each event occurred. In the same way any change to the pre-procedure diagnoses is also noted.

- The planned anesthetic procedures at a general level. For example, 'General Anesthesia (GA', 'Sedation', 'Local Anesthetic Block' or more specifically, for example, 'Epidural Block'

- the current date and location of the procedure(s)

Perform Staff, Patient and Equipment checks

Before commencing the procedure, it is necessary to check the following:

- WHO Safer Surgery checklist that establishes that members of the care team understand and agree upon the planned procedure(s) and also the role and responsibility of each member of the care team. This includes supervision relationships.

- the identity of the patient and that the surgical site has been correctly identified and marked

- condition and operation of major items of equipment, particularly the anesthetic machine/ventilator

- the availability of drugs and devices that may be required in an emergency such as opiate antagonists, defibrillating ECG machine, bag and mask

Most of these checks and activities are included the pre-procedural 'Timeout'

Record Equipment Used

It is necessary to record the equipment used in the procedure including:

- anesthetic machine/ventilator

- physiological monitors

- imaging equipment

- equipment to ensure homeostasis such as warming blankets

Attach monitors

Monitoring of vital signs and other parameters is absolutely required in intra-procedural anesthesia. The ways in which an association between a patient and a monitor (device), are initiated and managed are out-of-scope of this DAM but are described in detail in:

HL7 Version 3 Domain Analysis Model: Detailed Clinical Models for Medical Devices, Release 1 (HL7 Health Care Devices Work Group, March 2015)

Minimum and desirable, standards for monitoring in anesthesia have been defined by various national professional bodies and international organizations including AAGBI (UK and Eire), ANZCA (Australia and New Zealand), ASA (US), CAS (Canada), EBA (Europe) and WHO-WFSA (International). Based on the recommendations of these bodies, minimum requirements for monitoring in anesthesia can be defined as:

- Inspired O2

- Expired CO2

- Airway pressure

- Inspired and end tidal concentration of inhalational agents, when used

- Pulse oximeter

- Continuous ECG

- NIBP

- HR

- Temperature

- TOF measured using a Peripheral nerve stimulator when neuromuscular blocking agents are used

Increasingly data from patient monitors and other patient connected devices is collected automatically by the Anesthetic Information Management System (AIMS) which incorporates the data in the electronic record. The provenance of device data is again out-of-scope of this DAM but is covered in detail by publications of the Health Care Devices Work Group and in the ISO11073 standard.

Establish Vascular Access

Vascular access is essential in order to provide medications, fluid support and blood products. It is customary to establish at least peripheral access before induction and sometimes a central venous catheter and arterial line in addition. Pulmonary arterial catheters are used only in specialized procedures. Use cases are included in the model for CVC, A-line and PIVC.

## Intra-procedural overview

Figure 2 : Intra-procedural overview

| Actor | Description |
| --- | --- |
| Anesthesiologist | Anesthesiologists are physicians specializing in perioperative care, developing anesthetic plans, and the administration of anesthetics. The anesthesiologist takes part in all 3 phases of surgery: before, during, and after. |

Intra-Procedural anesthesia

This is the hub of all the procedural activity related to anesthesia. Many of the processes in intra-procedural anesthesia involve a relatively small number of common acts that are part of more complex ones. These include: Aseptic Technique, Site Preparation, Medication Administration, Patient Positioning, Seldinger Technique etc.

General Anesthesia

General anesthesia is the most frequent type of anesthesia for most surgical procedures. It minimizes stress on the patient and provides benefits for the surgeon. However, it is not uncommon for regional anesthesia to be used with just sedation. General anesthesia usually requires full respiratory support for the duration of the procedure.

Induction

Induction of general anesthesia is a significant milestone because beyond this point the patient generally requires constant respiratory support. Typically, it immediately precedes intubation and then the maintenance of anesthesia for the duration of the procedure.

Regional Anesthesia

Regional anesthesia includes many forms of nerve block including neuraxial blocks (spinal and epidural), plexus blocks and intra-venous regional anesthesia (IVRA). They are particularly used for procedures on the limbs, for obstetric analgesia and post-surgical pain control. Regional blocks are often accompanied by mild sedation and may also be used in parallel with general anesthesia.

Airway

During general anesthesia the patient usually ventilated mechanically through a supra-glottic or intra-glottic airway. This is placed immediately following intubation and is connected to the breathing circuit. The placement of this airway is a crucial stage in the procedure and there are many different airways devices and techniques for their placement.

Maintenance of Anesthesia

Maintenance of anesthesia may require adjustments to gas flows, concentration of inhalational anesthetic agents and ventilator settings as well as attention to regional blocks if these are used.

The physiological status of the patient, particularly, oxygen saturation and cardiovascular parameters, are continually monitored and are incorporated in the electronic anesthetic record.

Medication administrations, milestone and procedural events are also recorded by the anesthesiologist or other member of the care team.

End of Anesthesia / Emergence

At the end of anesthesia:

- Neuromuscular blockade is reversed

- Inhalational agents are stopped, and oxygen flow is increased

- The anesthesiologist assesses the degree of emergence and the competence of the patient to breath spontaneously

- The artificial airway, if present, is removed and the patient is provided with a face mask

- All being well, the patient is transferred to recovery

Paths at emergence from and transfer after anesthesia and surgery

Example 1:

Precondition:  Intubated, ventilated patient under general anesthesia with volatile agents or neuromuscular blockade , in whom it is planned to reverse anaesthesia and return to a ward area after surgery. This scenario would need to be adjusted to reflect steps taken in the case of an anesthetic conducted with total intravenous anaesthesia(TIVA)  or mix of TIVA and anesthetic gases

- As the end of surgery approaches, the depth of anesthesia is reduced by reducing concentrations of volatile agents and gases

- The depth of anesthesia may have been monitored using a depth of anesthesia monitor such as BIS, entropy or other form of processed EEG.

- The degree of residual neuromuscular blockade is assessed using a peripheral nerve stimulator

- If reversal of neuromuscular blockade is required, this will be done by an appropriate dose of an anticholinesterase (e.g. neostigmine) accompanied by an  antimuscarinic drug (atropine or glycopyrronium) to minimise vagal side effects such as bradycardia and excessive salivation.

- Neuromuscular blockade may also be reversed by newer drugs such as the cyclodextrin / sugammadex

- Adequate reversal of neuromuscular blockade needs to be present before anesthetic gases are discontinued and replaced with oxygen or air/oxygen mix

- The patient is assessed to ensure that there is adequate return of spontaneous respiratory activity

- All monitoring is continued during this period.

- Dependent on the nature of surgery, the patient and the preferences of the anesthesiologist, extubation of the trachea may be undertaken "deep" - i.e. anesthetic gases are continued until extubation has taken place and then discontinued. Alternatively, if a patient is extubated "awake", the anesthetic gases require to discontinued prior to extubation.

- Once extubated a face mask is placed on the patient. This may deliver 100% oxygen or an air oxygen mix, targeted to produce an approximate oxygen concentration in the gas mix

- The patient is observed to ensure adequate spontaneous respiration, circulatory stability and continued adequate reversal of neuromuscular blockade and muscle power.

- Once these conditions for transfer have been met, preparations for transfer to PACU are made:

  - surgical apparatus is removed, remaining drains and other apparatus (e.g. urinary catheters) are secured

  - physiological monitoring is continued

  - Patient protective equipment such as warming blankets are continued or applied as appropriate.

  - Monitor transfer block is removed from the OR Anesthesia workstation

  - Portable oxygen is to the patient face mask from a portable oxygen cylinder

  - The patient is transferred to a bed or trolley as appropriate with adequate personnel to manage the transfer and ensuring protection of all lines and drains.

- The patient is transferred to the PACU

- The anesthesiologist gives a summary and handover to the PACU nurse clearly detailing any specific instructions

- The patient is then supervised/monitored by the PACU nurse. This will include issues such as adequate pain control, stability of vital signs and any surgical issues such as unforeseen haemorrhage

- A deviation from anticipated recovery will be referred to the surgeon or anesthesiologist as appropriate

- When appropriate discharge criteria have been met, the patient will be discharged to the relevant ward area.

- Discharge will include a care plan including observation regime, continuing pain relief, continuing oxygen therapy and any other necessary instructions.

Example 2:

Preconditions:  Patient undergoing a spontaneously breathing anesthetic using volatile anesthesia with airway secured using a laryngeal mask airway (LMA).  The patient is planned to return to a surgical ward.

- Anesthesia is maintained towards the end of surgery

- preparations for transfer to PACU are made:

  - surgical apparatus is removed, remaining drains and other apparatus (e.g. urinary catheters) are secured

  - physiological monitoring is continued

  - Patient protective equipment such as warming blankets are continued or applied as appropriate.

  - Monitor transfer block is removed from the OR Anesthesia workstation

  - The patient is transferred to a bed or trolley as appropriate with adequate personnel to manage the transfer and ensuring protection of all lines and drains.

  - When ready for transfer, patient is disconnected from the anesthesia breathing circuit and the LMA is connected to a T-piece supplied with oxygen from a portable oxygen cylinder

- The patient is transferred to the PACU

- The anesthesiologist gives a summary and handover to the PACU nurse clearly detailing any specific instructions

- The patient is then supervised/monitored by the PACU nurse. This will include issues such as adequate pain control, stability of vital signs and any surgical issues such as unforeseen haemorrhage

- Any deviation from anticipated recovery will be referred to the surgeon or anesthesiologist as appropriate

- When appropriate discharge criteria have been met, the patient will be discharged to the relevant ward area.

- Discharge will include a care plan including observation regime, continuing pain relief, continuing oxygen therapy and any other necessary instructions.

Example 3

Preconditions: The patient is undergoing major surgery and postoperative care will consist of planned artificial ventilation in the Intensive Care Unit

- Anesthesia is maintained towards the end of surgery

- preparations for transfer to ITU are made:

  - surgical apparatus is removed, remaining drains and other apparatus (e.g. urinary catheters) are secured

  - physiological monitoring is continued

  - Patient protective equipment such as warming blankets are continued or applied as appropriate.

  - Monitor transfer block is removed from the OR Anesthesia workstation and inserted into a transport monitor

  - The patient is transferred to an ITU  bed or trolley as appropriate with adequate personnel to manage the transfer and ensuring protection of all lines and drains.

  - When ready for transfer, patient is disconnected from the anesthesia breathing circuit  and either artificially ventilated using a transport ventilator or manually ventilated using a self inflating bag supplied from an oxygen cylinder

- The patient is transferred to ITU by the anesthesiologist, associated anesthesia care professionals, nurses and porters as appropriate as defined by local convention

- The anesthesiologist, theatre nurse and possibly surgeon give a summary and handover to the ITU medical and nursing staff nurse clearly detailing any specific instructions

- Responsibility for the patient moves to the ITU clinical team once handover has been completed

### Major procedure example: Repair of aortic aneurism

This storyboard describes a major procedure, repair of aortic aneurysm. Model elements that are used are shown in the steps of the scenario

#### Basic Path

Pre-conditions

- Patient in the OR or Anesthetic Room

- All checks completed

Emergency Equipment and Drugs

All ORs should be equipped as standard with a suitable range of emergency equipment. This should, at minimum, meet the requirements of the WHO Generic Essential Emergency Equipment List.

All ORs should be equipped as standard with a full range of emergency drugs including those most used for anesthetic emergencies.

Procedural equipment and drugs

- Anesthetic machine/ventilator

- Cardio-pulmonary bypass machine

- Patient monitors

- Airway and vascular access kits

Post Conditions

Procedure complete, patient transferred to Cardiac Intensive Care Unit.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | <include> Site Preparation Nurse prepares dorsum of right hand using chlorhexadine gluconate 1% applied with a swab | Site Preparation |
| 2 | 1 % lidocaine is injected subcutaneously using a 27 gauge needle | Medication Administration |
| 3 | <include> Insertion of peripheral venous line A 14G I.V. catheter (Venflon Pro) is placed in the dorsum of the right hand by Anesthesiologist | Device Use |
| 4 | Anesthesiologist administers 1 mg midazolam via the IV line. | Medication Administration |
| 5 | Nurse prepares skin over the left radial artery using chlorhexadine gluconate 1% applied with a swab | Site Preparation |
| 6 | Anesthesiologist injects 1 % lidocaine subcutaneously using a 27 gauge needle over the left radial artery | DirectInjection |
| 7 | <include> Insertion of arterial line Anesthesiologist sites a 20G arterial line | Device Use |
| 8 | Anesthetic nurse connects ECG leads, pulse oximeter, blood pressure cuff etc. and the arterial line is connected by Anesthesiologist (arterial line set, transducer and pressure bag at 300 mgmHg). | Non-Device Use |
| 9 | Anesthesiologist connects arterial line (arterial line set, transducer and pressure bag at 300 mgmHg). | Procedure |
| 10 | Anesthesiologist asks the Patient to sit up on the edge of the bed gives 2mg Midazolam I.V. | Medication Administration |
| 11 | Anesthesiologist washes his hands and puts on gown, mask and sterile gloves. | Aseptic Technique |
| 12 | Anesthetic Nurse prepares skin over the throracic spinal region using chlorhexadine gluconate 1% applied with a swab and a fenestrated sterile drape is applied | Site Preparation |
| 13 | <include> Epidural The T9-T10 interspace is estimated by counting and an 18 gauge Tuohy needle is inserted using a midline approach. The epidural space is identified using loss of resistance (LOR) to saline. LOR is identified at 5 cm and an epidural catheter threaded to a total length from the skin of 8 cm. | Device Use |
| 14 | A T9-T10 epidural catheter is then placed by Anesthesiologist under aseptic conditions using a local anaesthetic. | Device Use |
| 15 | Anesthesiologist administers a test dose via the epidural catheter of 3ml of 0.5% bupivacaine with 1:100000 epinephrine. | Medication Administration |
| 16 | Anesthesiologist starts an infusion of 1000 ml Hartmann solution is started via the intravenous catheter | Medication Administration |
| 17 | Anesthetic Nurse asks Patient to resume a lying (supine) position | Positioning |
| 18 | Anesthetic Nurse places a warming blanket over Patient to maintain his core temperature. The temperature is set to 43°C | Non-Device Use |
| 19 | Anesthetic Nurse places a size 5 clear plastic face mask placed over Patient's face | Non-Device Use |
| 20 | <include> Pre-oxygenation | Procedure |
| 21 | Capnographic CO2 is sampled from the HME via a fine bore capnography tube at 120 mls/minute. | Procedure |
| 22 | <include> Induction Anesthesiologist induces anesthesia in the anaesthetic room/ OR Propofol emulsion 14 mls 1% (10mg/ml) | Medication Administration |
| 23 | <include> Endotracheal intubation using a layringoscope Anesthesiologist uses a laryngoscope with a Macintosh Size 4 blade to obtain a view of the glottis opening. Anesthesiologist notes the grade of Intubation (Grade I | Procedure |
| 24 | A cuffed oral endotracheal tube (PORTEX) (COETT) 8.5mm internal diameter is then placed through the larynx. The COETT is connected to the Bain circuit (ensuring HME in place via 15mm connector and flexible angle piece). Confirmation of ET tube placement is done using a stethoscope to auscultate the lung fields and the capnography trace is confirmed on the monitor. The ET tube is secured using a fabric tie. | Device Use |
| 25 | Anesthetist confirms correct ET tube placement using a stethoscope to auscultate breath sounds and capnography | Identification Of Anatomical Location |
| 26 | The Bain circuit is connected to a Nuffield Penlon ventilator and Pete Patient is mechanically ventilated at 10 breaths per minute with a tidal volume of 600mls | Respiratory Support |
| 27 | Pete Patient is kept asleep, using Isoflurane End Tidal 1%, 40% oxygen in air 2l/min, 6l/min carrier gas flow and airmix from the anaesthetic machine, using a circle system with CO2 absorber | Procedure |
| 28 | Tape is placed over Pete Patient’s eyes to avoid any corneal injury | Non-Device Use |
| 29 | The anaesthetic monitor records vital signs – heart rate, blood pressure (invasive and non-invasive, systolic, diastolic and mean), oxygen saturation (using a pulse oximeter), respiratory rate, end tidal CO2, temperature (core/nasopharyngeal), neuromuscular blockade (using a peripheral nerve stimulator, T4/T1 ratio e.g. 0.6) |  |
| 30 | <include> Central Venous Catheter At this time | Device Use |
| 31 | <include> Positioning Patient is placed in a head down position | Positioning |
| 32 | Anethesiologist uses an ultrasound machine to scan the right side of Pete Patient’s neck to check his anatomy |  |
| 33 | Anesthesiologist puts on a sterile gown and gloves | Aseptic Technique |
| 34 | Anesthesiologist prepares skin with 0.5% chlorhexidine | Site Preparation |
| 35 | Using the Seldinger technique – Anesthesiologist finds vein with needle then puts a guide wire through the needle and uses a dilator to make a hole big enough to put the Right Internal Jugular triple lumen (CVP) catheter (8 Gauge) through. The catheter is then stitched in place using a 2/0 silk on a straight needle. The CVP line is then flushed with saline 0.9% and is checked to see if all lumens are patent – blood can be easily aspirated. The line is then flushed again with 0.9%Saline. |  |
| 36 | Temperature is monitored using a nasal probe and values are recorded every 30 minutes for the remainder of the case |  |
| 37 | All monitoring is disconnected – arterial line, sats probe (Pulse oximeter), BP cuff, and ECG and warming blanket disconnected and removed.. The anaesthetic breathing circuit is disconnected. |  |
| 38 | Care Team transfer the Patient to the OR and place on the operating table. Several people help with the transfer using a ‘pat slide’ and ‘slidy sheet’. The monitoring remains connected as the anaesthetic monitor travels from the anaesthetic room with the Patient into the OR |  |
| 39 | Pete Patient is connected via a new circle circuit and the existing HME filter to the anaesthetic machine. Ventilation is commenced at 10 x 600 mls | Respiratory Support |
| 40 | Dr. Periop sets the anesthetic machine vaporizer to deliver Isoflurane 1% and sets fresh gas flow to deliver oxygen and air at 3L/minute | Medication Administration |
| 41 | Care Team place Patient on the operating table. | Procedure |
| 42 | Care Team place the Patient a supine position on the operating table. | Positioning |
| 43 | The monitoring is reconnected by ODP and Dr. Periop ECG, Pulse Oximeter, BP cuff, Arterial line, CVP monitoring line – (1st connection). All lines are arranged. |  |
| 44 | A fluid warmer [Arizant] and warming blanket are reconnected | Non-Device Use |
| 45 | Anesthesiologist inserts a nasal temperature probe and values are recorded every 30 minutes for the remainder of the case |  |
| 46 | Surgeon requests a nasogastric tube (16-18 French). Anesthesiologist passes a naso-gastric tube through Patient’s nose and advances it into his stomach. Correct placement is confirmed by aspiration of gastric contents and testing using a pH test strip | Device Use |
| 47 | Anesthesiologist administers 0.5mg of Metaraminol 0.5mg/ml i.v | Medication Administration |
| 48 | Anesthesiologist administers 10mls of 0.5% Bupivacaine and 100mcg of Fentanyl to Patient via the epidural catheter | Medication Administration |
| 49 | Care Team complete the 2nd stage of the WHO Safer Surgery checklist with input from all team members (time out) | Checks |
| 50 | Anesthetic Nurse cleans the abdomen with povidone iodine. Disposable paper drapes are used to create a sterile area | Site Preparation |
| 51 | Surgeon makes a transverse incision. He is assisted by a Registrar, Junior Surgeon and Anaesthesia Nurse(s) |  |
| 52 | Surgeon uses a trouser (bifurcated) graft to replace the damaged aorta and iliac vessels. [405522001] |  |
| 53 | Anesthesiologist maintains the level of anaesthesia with Isoflurane (around 1% ET conc.) + air. He also administers Fentanyl as required as 50 uG IV boluses | Medication Administration |
| 54 | Anesthesiologist administers Atracurium 10 mg. IV boluses at approximately 30 minute intervals | Medication Administration |
| 55 | <include> Train of Four | Procedure |
| 56 | Anesthesiologist places a Transthoracic echo probe into the Patient’s esophagus to monitor cardiac output a pulmonary artery catheter not being available |  |
| 57 | Anesthesiologist gives a dose of antibiotics (Cephazolin 2g IV) at Surgeon’s request. | Medication Administration |
| 58 | After infusing the antibiotic for 10 minutes, Dr. Periop notes that the patient has developed a rash, indicating allergy to the antibiotic | AllergyIntolerance |
| 59 | Anesthesiologist reduces the flow of fresh gas to 0.5l/min oxygen and 0.5l/min of air for the duration of the procedure. |  |
| 60 | An epidural infusion of 1% Bupivacaine and 2 microgrammes / ml fentanyl into epidural infusion catheter | Medication Administration |
| 61 | Prior to cross clamping of the aorta Surgeon asks Dr.Periop to give 5000units of Heparin IV which he gives via CVP line | Medication Administration |
| 62 | Surgeon completes the incision and initial dissection of the aneurysm sac, then places an aortic cross clamp. The time and event are noted in the anaesthetic record. |  |
| 63 | There are a couple of problems intra-operatively. Pete Patient loses a lot of blood 2000mls due to surgical difficulties and a Cell Saver is used to collect some of this blood 1500mls of which is processed and retransfused | Observation |
| 64 | He also receives a blood transfusion 2 units (approx 600mls) of packed red cells, a platelet 1 pool (Approx 150mls) of platelets and 4 unit of Fresh Frozen Plasma ( 600mls) IV ( before the end of the procedure. During the period of rapid bleeding and blood transfusion, coagulation performance is monitored using thromboelastography |  |
| 65 | Dr. Periop also administers 5 x1000mls bags of Hartmann’s solution (Crystalloid) at a variable rate 2000ml/hr minute to 100 ml/hr and he starts a Noradrenaline infusion 4mg in 50mls of 5% glucose with a rate of 5 mls /hour. during the operation to maintain Pete’s blood pressure. |  |
| 66 | Dr. Periop gives a blood transfusion 2 units (approx 600mls) of packed red cells, a platelet 1 pool (Approx 150mls) of platelets and 4 unit of Fresh Frozen Plasma ( 600mls) IV ( before the end of the procedure. During the period of rapid bleeding and blood transfusion, coagulation performance is monitored using thromboelastography |  |
| 67 | Anesthesiologist also starts a Norepinephrine infusion (up to 0.2 to 1.3ug/kg/min) by continuous infusion via the previously sited CVP line, using an infusion pump, to maintain normal blood pressure | Medication Administration |
| 68 | Surgeon comples replacement of the diseased vessels with an aortic bifurcation graft, the distal clamps and the aortic cross clamp are removed, with careful monitoring of the patient’s haemodynamic status. The times and events are noted in the anaesthetic record. |  |
| 69 | Towards the end of the operation a blood gas sample is taken from the arterial line. The sample is processed using a blood gas analyzer and the results noted: PaO2 13kPa (UK) or 140mmHg (US), pH 7.25, pCO2 6kPa or 49mmHg, base excess -6mmol/l, bicarbonate 21mmol/l. | Observation |
| 70 | The remainder of the procedure goes smoothly. Patient has no other complications. Surgeon completes the procedure. |  |
| 71 | At the end of the operation the volatile anaesthetic is discontinued and the patient is sedated using a continuous Propofol infusion (200mg/h or 70ug/kg/min) using a syringe driver or infusion pump and intermittent (as required by heart rate increases) intravenous bolus doses of 100ug Fentanyl | Medication Administration |
| 72 | Pete Patient is manually ventilated using an Ambu-Bag with oxygen enrichment (high flow oxygen, 10L/minute) before being placed in an ICU bed and transferred to ICU | Respiratory Support |
| 73 | Records completion of case |  |
| 74 | AIMS displays MDS (Minimum Data Set) omissions and prompts for completion or signed over-ride |  |
| 75 | Anesthesiologist indicates destination of patient after transfer |  |
| 76 | Anesthesiologist prints the anaesthetic record |  |

## Generic Clinical Procedures

Figure 3 : Generic clinical procedures in context of invasive device placement

### Positioning

Positioning the patient for a surgical procedure is a shared responsibility among the surgeon, the anesthesiologist, and the nurses in the operating room. The optimal position may require a compromise between the best position for surgical access and the position the patient can tolerate. The chosen position may result in physiologic changes and can result in soft tissue injury (e.g., nerve damage, pressure-induced injury or ulceration, or compartment syndrome). For this reason, it is important to protect vulnerable sites and pressure points with padding. From the anesthesia viewpoint, it is also necessary to support, stabilize and secure the position of the patient's arms using arm boards and straps to prevent injury and to consider the hemodynamic impact of position in managing the patient. Ideally, the position should provide:

- Optimal exposure of the surgical site

- Airway management, ventilation and monitoring access for anesthesia

- Physiologic safety for the patient

Of these, the first will normally have priority. It is important to record details of position(s) including timing in the anesthetic record even if these are not an anesthesia responsibility . The anesthesia care provider will normally be most concerned with the position of the head and neck both during the establishment of an artificial airway and subsequent access, and the position of one or both arms which are typically used for vascular access, monitoring of neuro-muscular blockade and the placement of the oxygen saturation sensor

#### Basic Path

- The basic body position, appropriate to an anesthetic or surgical procedure is established (with the cooperation of the patient, if conscious). Examples might be, sitting for a neuraxial block, supine position for an appendectomy. If the patient is on the table then the body position is established and secured before any table-tilt is applied.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Care team transfer patient from trolley onto operating table using a slide sheet |  |
| 2 | Basic position is established, with alignment of pelvis, spine and neck where possible | Positioning |
| 3 | Place rolls, wedges, foam to stablize chosen position and place padding under feet, head and identified pressure points | Positioning |
| 4 | Secure the body position with straps | Positioning |
| 5 | Tilt/configure operating table whilst ensuring existing lines and airway are not displaced | Positioning |
| 6 | Re-connect monitoring and check airway and lines |  |

#### Position arm with arm board

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Place arm on padded arm board and supinate to avoid damage to ulnar nerve |  |
| 2 | Support wrist with padding to avoid damage to median nerve |  |
| 3 | Set abduction angle which should not exceed 90 degrees |  |
| 4 | Cover arm with blanket |  |
| 5 | Secure arm with straps |  |

- Figure 4 : Positioning_ActivityGraph

### Aseptic Technique

Aseptic technique means using practices and procedures to prevent contamination from pathogens. It involves applying strict rules to minimize the risk of infection. Healthcare workers use aseptic technique in surgery rooms, clinics, outpatient care centers, and other health care settings.

It involves:

- Removal of jewelry and items such as wrist watches

- Hand washing. This is a basic component of aseptic technique and may need to be done more than once during preparation for some procedures

- Putting on Personal Protective Clothing (PPE) such as gloves, mask, hat, gown and/or apron or scrubs. For simple procedures, only gloves may be needed. For high-risk procedures, protective glasses or a face visor may be worn.

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Remove all jewellery |  |
| 2 | Check no artificial nails |  |
| 3 | Inspect hands for sores or abrasions |  |
| 4 | Clean hands with alcohol-based hand rub or soap and water |  |
| 5 | Turn on water |  |
| 6 | Apply microbial soap to hands |  |
| 7 | Keeping hands above elbows, start timing; scrub each side of each finger, between fingers, under each nail, and the back and front of hands for the recommended time, according to hospital |  |
| 8 | Scrub the arm Wash each side of the arm from wrist to elbow for one minute |  |
| 9 | Repeat the entire process with the other hand and forearm |  |
| 10 | With hands raised, rinse hands and arms by passing them through running water |  |
| 11 | Dry arms using a sterile towel |  |
| 12 | Apply personal protective equipment: e.g. gloves, cap, mask, gown and gloves. Assistance may be required to complete these actions in a sterile manner |  |
| 13 | Record aseptic precautions in procedural record |  |

### Site Preparation

Many procedures in anesthesiology involves the placement of an invasive device which introduces an immediate infection risk. It is therefore very important that the procedure site should be thoroughly cleaned and disinfected and that drapes should be used to ensure a sterile field.

#### Basic Path

The skin site should be thoroughly cleaned with one or more antiseptic solutions. If using an iodine-based solution in the context of a neuraxial block it is particularly important to allow full drying to avoid the risk of an inflammatory response. Either sprays or swabs may be used but swabs are preferred because the scrubbing action provides better cleansing.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Open procedural pack | Kit |
| 2 | Identify general procedural site | Identification Of Anatomical Location |
| 3 | Select an appropriate antiseptic agent | Medication |
| 4 | Using a swab, scrub the area using a circular motion and moving outwards from the center | Medication Administration |
| 5 | Allow the antiseptic solution to dry thoroughly |  |
| 6 | Drape the site to provide a sterile field |  |
| 7 | Drape the patient if required |  |

### Local Anesthetic Infiltration

Local cutaneous infiltration is the most commonly used anesthetic technique and involves direct injection into the area requiring anesthesia. It can be used for minor surgical procedures and also, commonly, to provide analgesia prior to inserting an invasive device such as an epidural needle. Common agents that are used are lidocaine, procaine and bupivacaine

Preconditions:

- Aseptic technique observed

- Site preparation performed

- Syringe loaded with local anesthetic and fitted with small bore needle (25G – 30G)

Post conditions

- Adequate analgesia

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Insert tip of needle into skin and inject a small quantity of local anesthetic | Medication Administration |
| 2 | Insert full length of needle along required path | Device Use |
| 3 | Aspirate to ensure the needle is not in a vessel |  |
| 4 | Inject local anesthetic whilst withdrawing the needle slowly | Medication Administration |

### Field Block

In this technique an initial injection of local anesthetic is made in the same way as for Local Anesthetic Infiltration. The needle is then rotated laterally, and a second injection is made so that the two paths diverge as ‘V’, away from the injection site. The operator then moves to the other side of the field, this time with the needle orientated towards the original injection site. Two injections are then made, mirroring the ones described above. At the conclusion of the process the four injection paths describe a diamond shape that encompasses the field.

Preconditions

- Aseptic technique observed

- Site preparation performed

- Syringe loaded with local anesthetic and fitted with small bore needle (25G – 30G)

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Include {Local Anesthetic Infiltration} |  |
| 2 | Rotate needle laterally, using same hole |  |
| 3 | Include {Local Anesthetic Infiltration} |  |
| 4 | Move to the other side of the field, this time with the needle orientated towards the original injection site. |  |
| 5 | Include {Local anesthetic infiltration} |  |
| 6 | Rotate needle laterally, using same hole |  |
| 7 | Include {Local anesthetic infiltration} |  |

### Device Guidance & Confirmation of Position

A few procedures are done without guidance (e.g. blind placement of a laryngeal mask) most use at least one type.

There are three purposes:

- accurate identification of the point where a device must be placed e.g. an electrode, or inserted e.g. a catheter

- guidance of an invasive device as it is moved towards its target

- confirmation of the final position of the device before it is used

### Anatomical Landmarks

When placing vascular lines and anesthetic catheters, the first step is to locate the correct entry point. One technique is to use so-called anatomical or surface landmarks that provide a reference from which the required site can be identified. This may be a single landmark but often two or more are needed for accurate location. A common example is the identification of the L3/L4 inter-vertebral space for a lumbar epidural block. Here, the left and right supra-iliac crests of the pelvis are found, and a line is drawn between them. This is known variously as the intercristal/Tuffier’s/Jacoby’s line. In around 75% of patients, the point where the line crosses the vertebrae usually corresponds to the L3/L4 space, in most others, the L2/L3. In some techniques, two lines are drawn, the first to provide a reference from which a second, perpendicular line projects towards the required site.

### Loss of resistance technique

Loss of Resistance (LOR) is a technique used in placing an epidural needle. A syringe, filled with air or saline, is attached to the hub of the needle and a constant pressure is maintained on the syringe plunger. As the needle traverse obstructing tissue, particularly the ligamentum flavum, there is no free passage for its contents and so a resistance is felt. As the tip of the needle breaks through into the epidural space, there is a sudden loss of resistance that signals the event.

### Imaging Guidance

Imaging can be used to visualize anatomical structures and the position of invasive devices in relation to them. Common modalities are ultrasound which is often used in regional block procedures and optical which is employed mainly in airway procedures (video-laryngoscopy and fiber-optic intubation). Flouroscopy is used infrequently although X-rays are used to confirm the correct placement of invasive devices e.g. a CXR to confirm the placement of a CVC.

### Nerve Stimulation

Nerve stimulation is frequently used in regional anesthesia techniques particularly, plexus blocks. An insulated needle is advanced towards the intended site and a current (normally around 0.5 mA) is applied. When the tip of the needle is close to the target nerve or nerve plexus a muscle that is innervated by that structure will twitch. Similarly. A saline filled epidural catheter can be used in a similar way.

### Seldinger Technique

Seldinger technique (named after its originator, Sven Seldinger) is used in a number of procedures where a device with a lumen needs to be introduced into a hollow structure such as a blood vessel or airway. In the context of anesthesia Seldinger technique is most often a component of the insertion of Central Venous Catheters (CVCs) and Peripherally Inserted Central Venous Catheters (PICCs) but may also be a component of other vascular access procedures as well as some airway procedures such as crycothyroidectomy. Initial access to the target is achieved using a needle/introducer through which a blunt-ended guidewire is passed.

The device, such as a catheter or tube, can then be railroaded over the wire, which guides it to its intended position.

A variant of this basic technique is referred to as ‘Modified Seldinger Technique in which a dilator with a peel-away sheath is used. Once the dilator has been placed using the guidewire, it and the wire are removed to leave the sheath in place. The procedural device e.g. catheter is then advanced directly through the sheath.

#### Basic Path

Pre-conditions

- The patient has been positioned as required

- Ultrasound machine is accessible and working

- Anatomy at planned insertion site has been evaluated using ultrasound

- Aseptic technique has been observed

- A sterile probe cover has been fitted over the ultrasound probe

- The site has been prepared with antiseptic swab and drapes applied appropriate to the procedure

- The catheter or other device has been got ready, lines have been flushed and ports capped

Summary

- Place guidewire through access needle

- Dilate the path along the guidewire

- Insert the catheter / tube along the guidewire

- Remove guidewire

Example: Seldinger technique used in insertion of vascular line.

The site is cleaned and prepared (see Site Preparation use case) and the vessel or cavity is identified. A hollow needle with syringe attached, is held at approximately 20o to the skin and inserted so as to enter the desired vessel. The syringe plunger is pulled back and blood is seen in the syringe as the needle pierces the vessel. Bright red, pulsatile, blood indicates arterial placement – this can be confirmed by blood gas analysis and pressure waveform. With the needle stabilized, the syringe is withdrawn, and the needle hub covered to prevent blood loss and entry of air into the vessel.

A guidewire is inserted through the hub of the needle and advanced into the vessel. The guidewire is advanced to the required length (this varies with the anatomical location and type of IV line being placed) and, with the wire stabilized, the needle is removed.

A guidewire, with depth markers, is inserted through the needle hub and the tip position is confirmed using ultrasound. The needle or cannula is then removed leaving the wire in place. A small incision is next made adjacent to the entry site of the wire and a dilator is advanced overt the wire to make a tract from the skin into the vessel or cavity. The dilator is then removed leaving the wire in place. The flushed catheter is advanced over the wire. In the case of a multi-lumen catheter the white and blue ports are capped while the brown port is left uncapped. The catheter is then advanced over the wire until the wire exits through the brown port. The guidewire is then removed. In Modified Seldinger technique the catheter is advanced directly through the peel-away sheath after removal of the wire and dilator. Each lumen is of the catheter is flushed. The catheter is secured, the site, dressed, and the catheter is labelled. Correct positioning is confirmed by imaging. The procedure is documented in accordance with local infection standards.

Example: Seldinger technique used in insertion of airway device in crycothyroidotomy

The cricothyroid membrane is palpated while immobilizing the larynx with the thumb and middle finger. An introducer needle is attached to a syringe, half-filled with water. A needle is inserted at a 30° (caudal) angle while applying negative pressure to the syringe. The appearance of bubbles in the syringe indicates that the needle has entered the trachea. At this point the syringe is removed leaving the needle in place. A guidewire is threaded through the needle into the trachea and the needle is removed, sliding over the guidewire. A 1-2 cm vertical incision is made down the cricothyroid membrane at the entry point of the guidewire. A tissue dilator fitted with a tracheostomy tube is now threaded over the wire and advanced into the skin incision along the curve of the dilator. Once in the trachea, the wire and dilator are removed leaving the tracheostomy tube in place. Finally, the tube cuff is inflated, and the tube is secured with ties or sutures.

In emergency, the dilator is dispensed with - a finger is used instead. Also, a gum elastic bougie may be used instead of a guidewire but other aspects of the procedure are the same.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Puncture vessel or cavity with needle |  |
| 2 | Pass guidewire through needle into vessel or cavity |  |
| 3 | Remove needle |  |
| 4 | Make skin nick next to wire. Optionally, enlarge with dilator |  |
| 5 | Advance procedural device over guidewire to target |  |
| 6 | Remove guidewire |  |
| 7 | Flush, secure and label procedural device |  |

#### Modified Seldinger

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Slide dilator and peel-away sheath over wire |  |
| 2 | Remove dilator and wire |  |
| 3 | Advance procedural device thru peel-away sheath to desired tip location |  |
| 4 | Remove peel-away sheath |  |

Figure 5 : Seldinger Technique_ActivityGraph

### Observation of Sampled Substance

When placing vascular lines and epidural catheters, useful information can be derived by examining aspirated substances. This is used more to confirm that the placement is correct than as a means to guide the procedure. For example, in the case of a vascular line, bright red, pulsatile blood indicates arterial placement and darker, non-pulsatile blood, venous placement. If fluid aspirated from an epidural catheter is warm or if it contains glucose and/or protein, then this suggests it is CSF and so the catheter is in the sub-arachnoid space. If it contains traces of blood/heme, then the catheter may have punctured a blood vessel. If no CO2 can be detected in expired gas, then this may suggest that an airway is in the esophagus and not the trachea.

### Test Dose

A test dose is sometimes used to assess the correct placement of an epidural catheter although the technique is used less commonly now than in the past. A small test dose e.g. 3ml lidocaine 1.5% with epinephrine 1:200,000, is injected via the epidural catheter. Two possible signs of incorrect placement may be seen. Noticeable loss of sensation, difficulty in raising the legs and a feeling of warmth may indicate that the catheter has punctured the dura and has been delivered intrathecally. A noticeable increase in heart rate (>20%) may indicate that the dose has been delivered into a blood vessel. In either case, this suggests that the catheter is not in the intended position i.e. the epidural space.

### Invasive Device Placement

An invasive device is any medical device that is introduced into the body, either through a break in the skin or an opening in the body. Examples are airway devices (such as endo-tracheal and endo-bronchial tubes, laryngeal mask airways and tracheostomy tubes), devices for vascular access (such as peripheral and central venous lines) and devices used in regional block procedures to administer local anesthetic agents, such as needles and catheters.

The placement of invasive devices is one of the most common categories of act during the anesthesia procedure. The details vary according to the precise context and type of device but there are generic features that can be characterized in a top-level use case.

The process of device placement often encompasses the placement of ancillary devices e.g. an epidural needle as an introducer for an epidural catheter or an intubating laryngeal mask as an introducer for an endotracheal tube. Seldinger technique, that is part of central line placements, involves the introduction of a wire to guide the progress of a catheter into the target vessel.

Whether, the device is the primary, procedural device , or an ancillary device that facilitates the placement, the general principles of insertion are the same.

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | <extend> Device Guidance & Confirmation of Position Identify entry site |  |
| 2 | <extend> Device Guidance & Confirmation of Position Insert and advance the device towards the target (using guidance if needed) |  |
| 3 | Withdraw the device slightly and adjust the approach if the entry is impeded |  |
| 4 | <extend> Device Guidance & Confirmation of Position Verify correct placement using guidance |  |
| 5 | Secure and label the device |  |
| 6 | Document the procedure |  |

### Tunneled placement

A tunneled placement is when an invasive device e.g. a catheter, tube or drain is tunneled under the skin to provide greater stability. It is an extension of the ordinary technique of line placement and is used when the line is intended for long term use.

In anesthesia the most common example of this kind of placement is a central venous line which is often placed in the internal jugular or femoral, vein. Some other names for tunneled CVC lines are Broviac, Hickman, Neostar, Leonard, and Groshong lines.

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Make incision at entry point (e.g. chest) |  |
| 2 | Make incision at exit point (e.g. neck) adjacent to catheter insertion point |  |
| 3 | Pass tunneller thru entry point, under skin, to emerge thru exit point | Device Use |
| 4 | Pass catheter through tunneller to project distally | Device Use |
| 5 | Remove tunneller | Device Use |

Figure 6 : Invasive Device Placement

## Mainstream Procedures

### Induction of general anesthesia

General anesthesia establishes a reversible state that includes:

- Hypnosis (sleep)

- Amnesia (no recall)

- Analgesia (pain relief)

- Akinesia (no voluntary movement)

- Autonomic and sensory block

The goals for induction of general anesthesia are to rapidly, safely, and pleasantly produce these four conditions while maintaining hemodynamic stability and ventilation.

General anesthesia can be induced using either intravenous agents, inhalational agents or both. It can also be maintained by the same methods as in total intravenous anesthesia. Neuromuscular blocking agents are usually given during induction which, together with unconsciousness, require the establishment of a reliable airway so induction of general anesthesia is followed immediately by the establishment of an airway. The commonest is an endo-tracheal tube but other devices such as a laryngeal mask may also be used.

Figure 7 : Induction of General Anesthesia

Pre-conditions

- Anesthesia machine checks complete and satisfactory

- Airway equipment checked and available

- Drugs for treatment of common complications and emergencies immediately available

- Standard monitoring applied and baseline value recorded

- Routinely administered anesthetic drugs prepared

- Intravenous access established

- Patient’s head in sniffing position and supported, bed elevated 20 to 30 degrees

- Preoxygenation performed

Emergency Equipment and Drugs

All ORs should be equipped as standard with a suitable range of emergency equipment. This should, at minimum, meet the requirements of the WHO Generic Essential Emergency Equipment List.

All ORs should be equipped as standard with a full range of emergency drugs including those most used for anesthetic emergencies.

Procedural equipment and drugs

- Anesthetic machine/ventilator + appropriate vaporizers and breathing circuit

- Laryngoscope (or alternative device for placing airway e.g. Glidescope, fiber-optic scope) as chosen by operator

- Laryngoscope blade (as chosen by operator)

- Other facilitating devices: stylets, bougies

- Intra-glottic and supra-glottic airway devices

- Drugs for induction as chosen for the procedure including inhalational agents, i.v. drugs such as propofol, muscle relaxants – these will normally have been prepared in advance (see Pre-conditions)

Post Conditions

- Patient unconscious and stable

- Anesthesia depth appropriate for surgery to begin

#### Basic Path

This describes a scenario for a routine epidural. Realistic examples of drugs, doses and descriptions of technique are included. There are a number of alternate paths, for example, where an initial attempt meets an obstruction and the needle is withdrawn slightly and directed slightly differently. In some cases, an attempt has to be abandoned and the needle withdrawn completely. Also, findings different from those in this basic path scenario could dictate action. Alternate and exception paths that address these issues will be described in a future release.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Anesthesiologist checks that after pre-oxygenation, the expiratory oxygen value exceeds 80%, |  |
| 2 | Anesthesiologist injects some lidocaine through the PIC line (to numb the vein) followed by propofol emulsion 14 mls 1% (10mg/ml), and atracurium 50 mg – all via PIC line. |  |
| 3 | Once the patient stops breathing, Anesthesiologist checks that it is possible to ventilate the patient and continues manual ventilation while muscle relaxant takes effect |  |
| 4 | Procedural Assistant tapes patient’s eyes |  |
| 5 | Anesthesiologist intubates the patient with an endotracheal tube, confirming correct placement by capnography (CO2 appearing in the expired gas), chest rise, good bilateral breath sounds, fogging of the endotracheal tube on expiration, and evident maintenance of oxygenation and then secures the endotracheal tube |  |
| 6 | Anesthesiologist turn on the ventilator, setting the mode to SIMV with Volume Control, ensuring first that the tidal volume is not too high, that the respiratory rate seems appropriate, and that the PEEP is appropriate |  |
| 7 | Anesthesiologist turns the vaporizer back on and selects appropriate tidal volume |  |
| 8 | Anesthesiologist starts the respiratory rate at about 15 or 16 breaths per minute adjusts this based on the end-tidal CO2 reading. |  |
| 9 | Anesthesiologist adjusts the anesthesia vaporizer based on the measurement of end-tidal values and EEG waveforms |  |
| 10 | Anesthesiologist sets PEEP at about 5 cmH2O at the start for normal-weight patients |  |

### Monitoring

Monitoring of vital signs and other parameters is absolutely required in intra-procedural anesthesia. The ways in which an association between a patient and a monitor (device), are initiated and managed are described in detail in:

HL7 Version 3 Domain Analysis Model: Detailed Clinical Models for Medical Devices, Release 1 (HL7 Health Care Devices Work Group, March 2015)

Minimum and desirable, standards for monitoring in anesthesia have been defined by various national professional bodies and international organizations including AAGBI (UK and Eire), ANZCA (Australia and New Zealand), ASA (US), CAS (Canada), EBA (Europe) and WHO-WFSA (International). Based on the recommendations of these bodies, minimum requirements for monitoring in anesthesia can be defined as:

- Inspired O2

- Expired CO2

- Airway pressure

- Inspired and end tidal concentration of inhalational agents, when used

- Pulse oximeter

- Continuous ECG

- NIBP

- HR

- Temperature

- TOF measured using a Peripheral nerve stimulator when neuromuscular blocking agents are used

Monitors are attached to the patient at the start of the procedure consistent with minimum standards and configured to measure and display desired parameters. The monitors may be disconnected briefly, for example during positioning and transfer. They are often disconnected permanently at the end of the procedure or a transport module may be left connected during transfer to the PACU.

Associations between the device and the patient and between the device and the AIMS are established.

During the procedure the anesthesiologist observes the monitors and may re-configure monitoring as required.

Data recorded from the monitor may have considerable complexity but for conventional clinical required information is:

identification/description of the parameter e.g. HR, NIBP (systolic)

- measurement method

- value and units of the measurement e.g. 70 bpm, 120 mmHg

- status of the measurement i.e. valid/invalid, artefact etc.

- time of the measurement

- id of the device providing the measurement

- site of the sensor(s)

- color and/or symbol used in displays/records for this parameter

It is assumed that details of the procedural, including the identity of the patient, are a known context.

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Attach sensor/probe to patient |  |
| 2 | Connect monitor |  |
| 3 | Configure monitor including clinician id, display parameters, alarms |  |
| 4 | Associate monitor with patient |  |
| 5 | Associate monitor with AIMS |  |
| 6 | Observe values on monitor (optionally also on AIMS) |  |
| 7 | Record values in manual anesthetic record if applicable |  |

### Train of Four (ToF) monitoring

The train of four technique is used to measure the degree of neuromuscular blockade. It uses a peripheral nerve stimulator to deliver a series of 4 stimuli which will cause a twitch in the innervated muscle under normal conditions. However, the use of neuromuscular blocking agents may attenuate or completely eliminate the twitch response. Further, a diminishing response may be observed to each stimulus in the sequence. As a rough guide: If less than 75% of receptors are blocked then 4 twitches may be seen, if only three twitches are seen then the percentage is > 75, if two, > 80, if three > 90 and if none, 100.

The most common nerve to use is the ulnar because the electrodes are placed on the wrist which is normally accessible, and the thumb twitch is easily seen. However, the tibial and facial nerves are also sometimes used.

A ToF ratio (ToFR) is calculated by dividing the amplitude of the fourth response by the amplitude of the first response.

#### Establish baseline

The patient’s arm is extended, palm up in a relaxed state and the skin is prepared with an alcohol wipe. The distal electrode is placed at the level of the wrist on the ulnar surface at the flexor crease, as close to the nerve as possible. The second electrode is placed 1-2 cm proximal to the first, parallel to the flexor carpi ulnaris tendon

Prior to the administration of neuromuscular blocking agents, it is helpful to apply the electrodes and establish a baseline supra-maximal stimulation point. To do this, the negative lead is attached to the distal electrode and the positive lead to the proximal electrode. The stimulator is then set to deliver four, 0.2 ms square wave pulses at an interval of 500 ms repeat every 10 seconds.

A low current ~ 10mA is selected and the train-of-four nerve stimulator is activated. The twitch responses to each of the train of four impulses may then be seen in the thumb. If four twitches are not seen or felt, increase the current is increased in increments, until four twitches are observed. The current is then increased by one further increment and if the response is the same as at the previous setting, the lower setting is recorded as the supra-maximal stimulation point (baseline)

#### Baseline

Attach the negative lead to the distal electrode and the positive lead to the proximal electrode

- Set the PNS to deliver four, 0.2 ms square wave pulses at an interval of 500 ms repeat every 10 seconds

- Set the current to the previously determined baseline value and activate the stimulator

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Prepare skin using appropriate alcohol based wipe |  |
| 2 | Apply first ECG electrode (distally) to ulnar nerve as shown in diagram and picture below |  |
| 3 | Apply second ECG electrode (proximally) to ulnar nerve as shown in picture above |  |
| 4 | Attach positive lead to proximal electrode, negative to distal electrode |  |
| 5 | Select low current ~ 10mA, and activate train-of-four nerve stimulator |  |
| 6 | The twitch responses to each of the train of four impulses may be seen in the thumb |  |
| 7 | If four twitches are not seen or felt, increase the current in increments, until four twitches are observed |  |
| 8 | Increase the current by one increment and if the response is the same as at the previous setting, record the lower setting as the supra-maximal stimulation point (baseline) |  |

Procedural

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Select already identified supra-maximal (basline current) |  |
| 2 | Observe and record twitches |  |

Figure 8 : Train of Four_ActivityGraph

## Vascular Access

Vascular access is a major part of intra-procedural anesthesia. It is used for:

- medication administration

- administration of fluids

- measurement of cardiovascular function

- acquiring samples

Procedures range from simple to complex and some carry significant risks of serious infection and/or injury. However, most procedures may be regarded as routine.

Figure 9 : Vascular access procedure

### Central Venous Catheter (CVC)

A central venous) line (CVC) is a catheter whose tip is located in the superior vena cava, in the right atrium, or in the inferior vena cava. These targets are also acquired by Peripherally Inserted Central Catheters (PICC). The main difference is that with a CVC, access is gained via a central vein such as the internal jugular whereas with a PICC it is via a peripheral vein (commonly, cephalic, basilic or brachial).

All CVCs are inserted using a guidewire (Seldinger technique. There are two versions of this technique, standard and modified. Both are described in the section on Seldinger Technique.

The CVC is a multi-lumen device and can be used for several purposes:

- Administering medications

- Administering fluids

- Measuring central venous pressure

- Obtaining blood samples to measure, for example, central venous oxygen saturation

Complete sterile precautions are observed throughout insertion of a CVC.

The main sequence is:

- Access central vein with needle

- Insert long (145 cm) guide wire

- Remove the needle

- Make a small incision at the entry point to widen it

- Insert a dilator over the guidewire and advance it into the vessel forming a tract from the skin to the vein

- Remove the dilator

- Thread the catheter over the wire and advance it along the wire to the target

- Remove the wire

Pre-conditions

- Ensure ultrasound machine is accessible and working

- Evaluate anatomy at planned insertion site using ultrasound

- Observe aseptic technique and use sterile probe cover for ultrasound

- Prepare the site with antiseptic swab

- Place a full surgical draper over the patient

- Prepare catheter, flush lines and cap ports

Emergency Equipment and Drugs

All ORs should be equipped as standard with a suitable range of emergency equipment. This should, at minimum, meet the requirements of the WHO Generic Essential Emergency Equipment List.

All ORs should be equipped as standard with a full range of emergency drugs including those most used for anesthetic emergencies.

Procedural equipment and drugs

- Ultrasound machine

- CVC kit containing catheter, guidewire scalpel etc.

Post Conditions

- Catheter secured and lumens patent

- Correct position confirmed by CXR

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Identify the needle insertion site that will allow cannulation of the desired vein. |  |
| 2 | Anesthetize the area over the insertion site. |  |
| 3 | Insert and advance the needle while continuously aspirating. |  |
| 4 | Follow the needle tip into the vessel using ultrasound guidance. |  |
| 5 | Successfully cannulate the vessel and ensure free return of dark red, non-pulsatile blood. |  |
| 6 | Stabilize the needle while disconnecting the syringe. |  |
| 7 | Insert guidewire through the needle to a depth of 20 cm. |  |
| 8 | Remove needle while maintaining control of the guidewire. |  |
| 9 | Confirm guidewire is in the vessel using ultrasound. |  |
| 10 | Make small skin incision adjacent to the guidewire. |  |
| 11 | Advance dilator over the guidewire and into skin with a twisting motion while maintaining control of the guidewire. |  |
| 12 | Ensure the guidewire moves freely within the dilator, then remove dilator. |  |
| 13 | Advance catheter over the guidewire and into the vessel to the appropriate depth for the selected insertion site. |  |
| 14 | Remove guidewire and keep the brown port covered to prevent air entry until a cap can be placed on the brown port. |  |
| 15 | Aspirate and flush each lumen. |  |
| 16 | Suture the line in place at the blue/white clips and the triangular holder. |  |
| 17 | Add biopatch and place a sterile dressing. |  |
| 18 | Obtain CXR following the procedure. |  |

Figure 10 : Central Venous Catheter insertion

### Peripheral Venous Catheter (PVC)

A peripheral intravenous line (PIV) is the most commonly used method for the routine deliver of medications and fluids. It involves the introduction of a catheter into a peripheral vein, commonly on the lower arm, or dorsum of the hand and can usually be done without imaging guidance. The catheter may have up to three lumens so that more than one substance can be administered concurrently.

Pre-conditions

- Aseptic technique observed

- Materials for the procedure are at hand including a tourniquet

Emergency Equipment and Drugs

All ORs should be equipped as standard with a suitable range of emergency equipment. This should, at minimum, meet the requirements of the WHO Generic Essential Emergency Equipment List.

All ORs should be equipped as standard with a full range of emergency drugs including those most used for anesthetic emergencies.

Procedural equipment and drugs

- TourniquetSyringe

- Sterile saline

- PIV needle and catheter

Post Conditions

- Catheter secured and lumen(s) patent

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Place tourniquet on limb above intended insertion site |  |
| 2 | Locate vein |  |
| 3 | Prepare site with antiseptic |  |
| 4 | Insert needle with catheter at a 25 – 30 degree angle |  |
| 5 | Watch for flashback of blood in catheter as access is obtained |  |
| 6 | Decrease the angle with the skin to about 5 degrees |  |
| 7 | Advance the needle with catheter until just short of the skin, then slide the catheter off the needle, stabilizing the needle as this is done. Advance the catheter fully into the vein |  |
| 8 | Apply pressure above the insertion site and remove the needle |  |
| 9 | Attach the connector to the catheter |  |
| 10 | Secure the catheter with tape and an occlusive transparent dressing |  |
| 11 | Flush the catheter with saline and clamp |  |

### Arterial Line (A-line)

An arterial line (also art-line or A-line) is a thin catheter inserted into an artery. It is most commonly used in intensive care medicine and anesthesia to monitor blood pressure directly and in real-time (rather than by intermittent and indirect measurement) and to obtain samples for arterial blood gas analysis. Arterial lines are generally not used to administer medication, since many injectable drugs may lead to serious tissue damage and even require amputation of the limb if administered into an artery rather than a vein.

An arterial line is usually inserted into the radial artery in the wrist, but can also be inserted into the brachial artery at the elbow, into the femoral artery in the groin, into the dorsalis pedis artery in the foot, or into the ulnar artery in the wrist. (Wikipedia)

Pre-conditions

External anatomy of the selected insertion site has been assessed

- Patient assessed for ischemic complications

- Patient's wrist dorsiflexed and secured to arm board

- Full aseptic technique observed

- Site prepared with antiseptic swabs

- Drapes applied to create sterile field

Emergency Equipment and Drugs

All ORs should be equipped as standard with a suitable range of emergency equipment. This should, at minimum, meet the requirements of the WHO Generic Essential Emergency Equipment List.

All ORs should be equipped as standard with a full range of emergency drugs including those most used for anesthetic emergencies.

Post conditions:

- Arterial line in correct position and lumen(s) patent

#### Basic Path

This scenario illustrates the basic path for a 'blind' insertion (not using ultrasound guidance) of a line in the radial artery. A scenario for ultrasound guided arterial line insertion will be added in a future release.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Palpate radial artery to identify its location and trajectory |  |
| 2 | Insert needle at 30 degree angle in line with path of vessel |  |
| 3 | Cannulate vessel |  |
| 4 | Wait for free return of pulsatile blood |  |
| 5 | Insert guidewire through the needle to a depth of approximately 10 cm. |  |
| 6 | Remove needle while maintaining control of the guidewire. |  |
| 7 | Make small skin incision adjacent to the guidewire. |  |
| 8 | Advance catheter over the guidewire and into skin while maintaining control of the guidewire. |  |
| 9 | Remove the guidewire and cover catheter with fingertip. |  |
| 10 | Connect arterial line pressure transducer. |  |
| 11 | Confirm pressure trace on monitor |  |
| 12 | Suture the line in place. |  |

## Airway Management

### Routine laryngoscopic endotracheal intubation

Figure 11 : Intubation

| Actor | Description |
| --- | --- |
| Anesthesiologist | Anesthesiologists are physicians specializing in perioperative care, developing anesthetic plans, and the administration of anesthetics. The anesthesiologist takes part in all 3 phases of surgery: before, during, and after. |
| Anesthetic Machine/Ventilator |  |
| Patient | A person who is the recipient of healthcare services. In this use case, a Patient is someone scheduled for a surgery or procedure that requires the provision of anesthesia services, who is undergoing pre-anesthesia evaluation. |
| Patient Monitor | Patient monitors measure, record, distribute and display combinations of biometric values such as heart rate, SPO2, blood pressure, temperature and more. High-capability, multi-function monitors are typically used in hospitals and clinics to ensure a high-level of quality patient care. |

Tracheal intubation using a laryngoscope to facilitate the introduction of an oral endo-tracheal tube is the commonest airway procedure in anesthesia.

#### Basic Path

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Assemble and check equipment |  |
| 2 | Pre-oxygenate patient | Medication Administration |
| 3 | Position patient | Positioning |
| 4 | Insert tube | Device Placement |
| 5 | Ventilate patient |  |
| 6 | Confirm position of tube | Identification Of Anatomical Location |
| 7 | Secure tube and ventilate |  |

#### Inserted too far

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Withdraw tube 2 -3 cm |  |

#### Wrong position

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Remove tube completely |  |
| 2 | Document position |  |
| 3 | Increment attempt count |  |

Figure 12 : Endotracheal intubation using a layringoscope_ActivityGraph

## Regional Anesthesia

### Epidural

An epidural is a local anesthetic neuraxial block. Neuraxial anesthesia and analgesia techniques include spinal, epidural, and combined spinal epidural (CSE). An epidural can be used to provide analgesia for surgery, either on its own or as an adjunct to general anesthesia, to provide pain relief during and after surgery. Neuraxial anesthesia is performed by placing a needle between vertebrae and injecting medication into the epidural space (for epidural anesthesia) or the subarachnoid space (for spinal anesthesia) either directly or via a catheter placed through the needle. Catheter-based neuraxial anesthesia (epidural, CSE, and continuous spinal) allows prolonged pain control and convenient titration of the dose. ‘Single-shot’ spinal or epidural anesthesia is limited to the duration of action of the injected drug. Use of a single shot is more common in the case of spinal blocks, a continuous block using a catheter is more common with epidurals.

In the conventional procedure, the point of injection is determined by feeling for specific bony landmarks on the spine and pelvis. A small volume of local anesthetic is injected into the skin and interspinous ligament. A needle is advanced slowly through the interspinous ligament until resistance is no longer felt to the attempted injection of air or saline, indicating that the tip of the needle is in the epidural space (the loss-of-resistance technique). A catheter is then threaded through the needle into the epidural space, the needle is removed, and the catheter is secured.

A number of approaches can be used e.g. midline, para-median, Taylor (modified paramedian) and caudal and it may be necessary to adjust the approach if difficulty in passing the needle is encountered. Occasionally, the needle must be withdrawn completely, and a new attempt made either at the same, or a different place.

Ultrasound guidance may be used to facilitate catheterization of the epidural space. One method is to use real-time ultrasound imaging to observe passage of the needle towards and into the epidural space. The second method (prepuncture ultrasound) is to use ultrasound as a guide to the conventional technique. In this method an initial ultrasound scan of the patient's lumbar spine is performed to locate the midline and the middle of an interspinous space; the position of each is marked on the skin. The depth of the epidural space is also determined from the ultrasound scan. Catheterization is then done in the conventional way, but with the skin markings as an additional guide. In both methods, 'loss of resistance' during passage of the needle remains an added safeguard against dural puncture. As with the conventional procedure, ultrasound-guided catheterization of the epidural space is performed under sterile conditions.

It is also possible to confirm the position of the needle by use of an insulated needle connected to a peripheral nerve stimulator although this is less common.

Procedural complications include:

- paresthesia

- dural puncture

- intrathecal injection of anesthetic

- epidural vein cannulation

Figure 13 : Epidural block

Pre-conditions

- Anesthesiologist has reviewed the patient’s medical history and discussed it with the patient

- Anesthesiologist has reviewed lab results

- Anesthesiologist has determined that there are absolute contra-indications and that the procedure is indicated

- Anesthesiologist has discussed risks and benefits with the patient and has obtained consent

- Emergency equipment and drugs have been assembled and are available for immediate use

- Standard monitoring is applied

- Peripheral IV access is established

- Oxygenation (with nasal prongs) if required

Emergency Equipment Emergency Drugs

- Ambu bag with mask Ephedrine

- Oxygen source Phenylephrine

- Oral and nasal airways Epinephrine

- Laryngoscope handles and blades Atropine

- Endotracheal tubes Sedative/hypnotic

- Eschmann stylet/bougie 20% lipid emulsion

- Syringes and needles Succinylcholine

Procedural equipment and drugs

Standard kit that typically includes the following: a sterile drape; prep swabs; 4 × 4 gauze sponges; a paper towel; povidone-iodine solution; an ampoule of 0.9% preservative-free sodium chloride; a 5-mL ampoule of 1.5% lidocaine with epinephrine 1:200,000; a 5-mL ampoule of 1% lidocaine for skin infiltration; a filtering device (needle or straw); a bacterial filter; needles and syringes of various sizes; a styletted epidural needle with cm markings; a 5- or 10-mL glass or plastic LOR syringe (either Luer lock or Luer slip); a catheter connector securing device; an epidural catheter with centimeter gradations and a connector/adapter; a thread assist device (TAD); a needle guard for sharps disposal; and labels.

In an adult epidural kit, the epidural needle is typically 17 or 18 gauge and 9 cm (roughly 3.5 in.) in length, with surface markings at 1-cm intervals.

Ultrasound monitor and probe with sterile probe cover (if required)

Post Conditions

- Catheter successfully placed and position verified

- Patient comfortable

- Adequate analgesia

- Procedure documented

#### Basic Path

This describes induction of general anesthesia using a mix of inhalational and intravenous agents. In fact, induction can be done using either on its own, the procedural steps are essentially unchanged. Realistic values for drugs doses and ventilator settings are included as examples. Alternate and exception paths will be described in a future release.

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | ACSP places patient in a sitting position (jackknife, lateral decubitus or prone might also be used) |  |
| 2 | AASP performs Aseptic Technique |  |
| 3 | ACSP identifies insertion site using anatomical landmarks and/or ultrasound |  |
| 4 | ACSP prepares insertion site using chlorhexidine swab and injects 1% lidocaine around insertion site |  |
| 5 | Nurse administers 1 mg midazolam and 50 mcg fentanyl via IV line at request of AASP |  |
| 6 | ACSP inserts 17G Tuohy needle into intra-spinous ligament using a midline approach (or para-median, modified paramedian) |  |
| 7 | ACSP removes the stylet and attaches a loss-of-resistance syringe containing 8 normal saline to the Tuohy needle |  |
| 8 | ACSP advances the needle in small steps, at the same time tapping the syringe plunger |  |
| 9 | After the needle has advanced 6 cm resistance to the syringe plunger suddenly drops indicating that the needle has entered the epidural space |  |
| 10 | ACSP removes syringe from needle |  |
| 11 | ACSP passes a 19G, wire-reinforced, end-hole catheter through the Tuohy needle to a depth of 16cm, then withdraws the needle over the catheter. |  |
| 12 | The marks on the catheter indicate that from the tip to the skin is 12 cm which, less the depth to space measurement of 6 cm, previously noted suggests that 6 cm of the catheter is in the epidural space. The AASP withdraws the catheter by 1 cm leaving 5 cm in the space. |  |
| 13 | ACSP checks to see if there is any fluid returning passively through the catheter (which might indicate intra-thecal placement). There is not. |  |
| 14 | ACSP attaches empty 3 ml syringe to catheter and aspirates gently. No blood is seen (which might indicate vascular puncture). There is, however, clear fluid. This is tested with a glucose strip but no glucose is registered showing the fluid is saline not CSF (other tests might include protein and pH) |  |
| 15 | ACSP draws up 3ml of lidocaine with 1:200,000 epinephrine in a 5ml syringe, attaches it to the connector and injects it via the catheter. There is no profound numbness (which would suggest intra-thecal placement) and no change in HR (which might indicate the catheter is in a vessel, previously tested by negative blood in aspirate) |  |
| 16 | ACSP puts liquid adhesive on patient’s back and secures the catheter in place with transparent adhesive dressing and tape |  |
| 17 | ACSP checks dermatome levels for change in sensation using alcohol swab and toothpick. None are observed. |  |
| 18 | ACSP now gives 7 ml lidocaine with 1:200,000 epinephrine, waits 10 minutes and repeats the test. A dermatomal change in sensation is identified. The dermatome levels at both sides are recorded. |  |

Figure 14 - Epidural block

## Respiratory Support

### Pre-oxygenation

Prior to the induction of anesthesia, or any other situation where the airway is to be controlled, preoxygenation is used to increase the safe apneic period (SAP).

Preoxygenation is the replacement of the nitrogen volume of the lung with oxygen, as well as maximizing the dissolved oxygen in the body tissues, and raising the partial pressure of oxygen in the blood.

#### Basic Path

Attach standard monitoring and position the patient ‘head up’ at 30 degrees (or more), with the auditory meatus above the jugular notch. Ensure that the airway is patent then, if apneic oxygenation will be used once the patient is sedated, place a standard nasal cannula (delivering 15 L/min oxygen) prior to placement of the preoxygenation device.

Choose a preoxygenation device based on the patient’s SpO2:

if SpO2 >95% use: bag-valve-mask (BVM) with PEEP valve and a good seal at 15+ L/min O2 or more, or non-rebreather (NRB) mask and a good seal at 15 L/min O2 or more

if SpO2 <95% use: BVM with PEEP valve and a good seal at 15+ L/min O2 or more

if adequate respiratory drive, preoxygenate by at least 3 minutes of tidal ventilations, or 8 breaths with full inspiration/ expiration to achieve vital capacity in <60 seconds (requires patient cooperation)

if inadequate respiratory drive, preoxygenate by positive pressure ventilation (e.g. assisted breaths with BVM or NIV) at 15+ L/min O2 or more

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | <include> Standard Monitoring |  |
| 2 | <include> Positioning |  |
| 3 | Ensure patient has patent airway |  |
| 4 | Set oxygen 100%, = 15 l/min |  |
| 5 | Attach mask to breathing circuit |  |
| 6 | Place mask on patient |  |
| 7 | Ask patient to take 8 vital capacity breaths or ventilate with BVM |  |

#### Apnoeic ventilation after sedation

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Place nasal cannula |  |

#### Inadequate respiratory drive

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Positive pressure ventilation / assisted breaths |  |

#### SpO2 < 95%

| # | Description of step | Resources used |
| --- | --- | --- |
| 1 | Use BVM with PEEP |  |

Figure 15 : Pre-oxygenation (Activity)

# Information component

### Modes of Ventilation

The terms used in this section are appropriate for the majority of ventilators in current use. For special applications, some manufacturers now offer ventilators with modes appropriate for those applications e.g. Bi-level AV and APRV (Airway Pressure Release Ventilation). Such modes will often use labeling terminology that is more appropriate to the ways that these modes function.

Figure 16 Modes of ventilation

#### VentilationMode (Class)

A ventilation-mode is the specified manner in which a ventilator performs its ventilatory function when connected to a patient (ISO 19223)

The ventilation-mode is a compound of the ventilation-pattern and the inflation-type(s). The syntax is VP1-IT1\IT2\IT3, where 'VP' is 'ventilation-pattern' and 'IT' is inflation-type. There is always one ventilation-pattern per mode but there can be between 0 and 3 inflation-types (in practice, typically in anesthesia, between 0 and 2 inflation-types are used). In the preparation of ISO 19223 it was found to be helpful to consider each ventilation-mode as falling into 1 of 4 groups, each with common core ventilation-pattern characteristics, as follows:

Group1: group of ventilation-modes sharing ventilation-patterns in which only one inflation-type can be selected at a time, this being assured to be initiated at least at the set rate (ISO 19223)

Group 2: group of ventilation-modes sharing ventilation-patterns in which an inflation-type is selected to be initiated at the set rate; between these assured inflations spontaneous breathing is possible, which can be either unassisted or supported by a selected support inflation-type (ISO 19223)

Group 3: group of ventilation-modes sharing ventilation-patterns in which every patient-trigger event initiates a selected inflation-type; if a patient-trigger event does not occur within a set time interval a second selected inflation-type is initiated by the ventilator (ISO 19223)

Group 4: group of ventilation-modes sharing ventilation-patterns that enable continuous unrestricted breathing or continuous supported breathing, always with a constant baseline airway pressure at the set BAP level (ISO 19223)

Note:   / = .XOR, \ = .AND

Examples of modes that fall into each of these 4 groups are listed in the table below.

| Group 1 | Group 2 | Group 3 | Group4 |
| --- | --- | --- | --- |
| CMV-VC | IMV-PC\PS | S/T-PS/VC | CSV-PS |
| CMV-PC | IMV-vtPC\PS | S/T-PS/PC(q) | CSV-vtPS |
| CMV-vtPC | SIMV-VC\PS | S/T-PS/vtPC | CSV-ES |
| A/C-VC | SIMV-PC\PS |  | CPAP1 |
| A/C-PC | SIMV-vtPC\PS |  |  |
| A/C-vtPC |  |  |  |

1 In the absence of either an assured inflation-type or a support inflation-type, as a setting, the ventilation-mode becomes CPAP. CSV-PS ventilation modes do not become CPAP when the PS is set to zero.

Three of these 4 Groups of ventilation-modes can be further divided into two subsets, identified as a and b. [For further information, see ISO 19223 or search for a specific term on the ISO Online Browsing Platform (https://www.iso.org/obp/ui/#iso:std:iso:19223:ed-1:v1:en)].

Ventilation-patterns and inflation-types are described in the notes for the corresponding classes. Note that where there is a combination of 1 pattern and 2 inflation-types e.g. in the case of mode, S/T-PS/VC, the ventilation-mode effectively switches between two types, depending upon whether the spontaneous breath rate is above or below the set rate. Therefore:

If the patient is spontaneously breathing at > set rate then the ventilator functions as if it were in the ventilation-mode CSV-PS (Ventilation-pattern: CSV, Inflation-type: Pressure-support)

If the patient breathes at < the set rate then the ventilator functions as if were in ventilation-mode IMV-VC (Ventilation-pattern: IMV, Inflation-type: Volume control)

The terms used in this section are appropriate for the majority of ventilators in current use. For certain applications, some manufacturers now offer ventilation-modes with particular characteristics or which are intended for use with unusual settings, such that they are considered to justify an alternative ventilation-mode name. Such modes, e.g. Bi-level AV and APRV (Airway Pressure Release Ventilation), will often use labelling terminology that is more appropriate for those settings.

In ISO 19223, specific classification schemes have been introduced for ventilation-modes, inflation-types, ventilation-patterns and ventilation-mode adjuncts. These serve, not only to facilitate their identification and relationships, but as a basis for systematic naming and coding.

| Attribute | Notes |
| --- | --- |
| mode CodeableConcept | This is the compound mode description e.g. S/T-PS/PC(q) |

#### VentModeSetting (Class)

Ventilation-mode settings can include:

- Set rate

- I:E Ratio

- Inspiratory time fraction

- BAP (set PEEP)

- Tidal volume

- Minute volume

- Pressure limits

- Expiratory time

- FiO2

- High-airway-pressure relief limit

- High-airway-pressure termination limit

The ventilation-mode defines the settings required in the context of a selected ventilation-pattern.

If a BAP setting is used, the resulting baseline airway pressure becomes the target level for PEEP so a separate setting for PEEP is not required. On ventilators without a BAP setting, the intended PEEP level will be determined by a dedicated setting.

| Attribute | Notes |
| --- | --- |
| ventModeSettingValue Quantity |  |

#### VentilationPattern (Class)

A ventilation-pattern is a specified temporal pattern of sequenced interactions between a ventilator and the patient, including which, when and by what, selected inflation-types are initiated. Its primary purpose is to provide the sequencing instructions required by the ventilation-mode function. It also determines which inflation-types can be included into that sequence and for what purpose.

Ventilation-patterns are invariably too complex to fully classify using a few simple words. It has therefore become customary to identify them by means of short descriptive names or by easily remembered associated abbreviations or acronyms although, because of a lack of standardization, the meanings attached to these identifiers have become very variable. However, in most cases the name has not been restricted to use with a specific inflation-type and it is therefore equally suitable to name just the ventilation-pattern. In ISO 19223, well established, non-proprietary ventilation-mode acronyms, or their names, have been adopted wherever possible as the generic names for the principle ventilation-patterns associated with them.

A specified ventilation-pattern, together with a specified inflation-type(s) and the type of any ventilation-mode adjunct, constitutes the core, systematic name of any ventilation-mode used for positive-pressure ventilation.

Although the ventilation-pattern determines the start of a respiratory cycle and of the duration of those subsequent phases not determined by an inflation-type, it does not control that duration directly. It determines them by instructing that the next event is to be after the phase duration resulting from the ventilation-mode settings that determine the duration of that phase, such as the set rate, the phase time ratio, or the next patient- trigger event.

There are no settings for a ventilation-pattern; It is independent of the inflation-type(s) selected, and their settings, as well as the set values for the ventilation-mode parameters.

Defined patterns are:

| Full name | Abbreviation |
| --- | --- |
| intermittent mandatory ventilation | IMV |
| synchronized intermittent mandatory ventilation | SIM |
| spontaneous/timed ventilation | S/T |
| continuous spontaneous ventilation | CSV |
| continuous positive airway pressure | CPAP |

| Attribute | Notes |
| --- | --- |
| ventPatternType CodeableConcept |  |

#### VentilationModeAdjunct (Class)

A ventilation-mode adjunct is a function of a ventilator, which provides ancillary actions to a ventilation-mode

Examples: ACAP, ETT compensation, leak compensation, breathing-system compliance compensation.

The principle ventilation-mode adjunct introduced in ISO 19223 is ACAP (assured constant airway pressure), which is defined as a built-in adjunct that enables unrestricted breathing by acting to maintain the airway pressure at its set value, irrespective of inspiratory or expiratory flows, or specified permitted leakage, whenever it is intended to be at a constant level.

A ventilation-mode adjunct can be, for example, permanently active, operator-initiated or configured by the responsible organization or manufacturer to become active when a particular ventilation-mode is selected. Additional properties that are selectable by the operator in order to modify a specific ventilation-pattern or inflation-type are considered to be optional variations, not ventilation-mode adjuncts.

The ancillary actions are adjunctive in that they only make changes that supplement the intended effect of the selected ventilation-modes and their settings.

With the selection of certain ventilation-modes the ACAP adjunct might be appropriately restricted to just one of the two basic phases. It will then be labelled as ACAP (high) or ACAP(low).

There are no settings for ventilation-mode adjuncts.

| Attribute | Notes |
| --- | --- |
| adjunctDescription CodeableConcept | adjunctDescription Enumerations: ACAP ACAP (high) ACAP (low) |

#### PatientTriggerEvent (Class)

Patient-trigger event (Synonym: trigger-event)

A patient-trigger event is a signal used to initiate an inflation, resulting from a measured value(s) of a parameter(s) that can be attributed to the patient reaching a threshold value

The detection of a measured value(s) of a parameter(s) reaching a set threshold value is the function of the trigger

#### Trigger (Class)

A trigger is a function that detects that the measured value(s) of a parameter(s) that can be attributed to the patient has reached the threshold value required to initiate another event

This is the definition of the basic concept of a trigger function, as the term is used in ISO 19223. It differentiates that class of inflation initiation signals that depend upon the attainment of a threshold by the measured value of a patient parameter, and hence involve a sensitivity setting, from those generated directly by the operation of simple timed, manual or remote, binary-switch functions.

The allowance of possible plural values and parameters is because some such functions might monitor the value of more than one parameter and make use of dedicated detection algorithms in order that sensitive settings can be used with a minimum risk of auto-triggering.

Even when the value of more than one parameter is used in a detection algorithm, it is common practice to include the dominant parameter in its designation, e.g., flow trigger, pressure trigger.

| Attribute | Notes |
| --- | --- |
| triggerType enum [1...1] | The type of trigger, which can include: flow pressure electromyogram neural complex computed chest-wall motion transthoracic impedance. |
| triggerParameterValue Quantity [1...1] | Flow: L/min Pressure: cmH2O Complex computed and others: Simple sensitivity scale, e.g., Expanding-width scale; Linearly-numbered scale |

#### InflationType (Class)

An inflation-type is a ventilator action intended to increase the volume of gas in the lungs by the application of an elevated pressure waveform to the patient-connection port until a specified termination criterion

It is characterized by the delivered-waveform pattern, following initiation, and the primary termination criterion

It is typically designated by its property-class common name, such as VC, PC, PS, vtPC, etc.

It may be more precisely designated using a systematic coding scheme that adds designators of additional properties to the common name abbreviations

When selected for a specific purpose it may be given a purpose-class name. For example, a VC inflation-type may be selected to be the ‘assured inflation-type’

One of one or more selected inflation-types are initiated by the VentilationMode, after an elapsed time or in response to a patient-trigger event, in accordance with the sequencing instructions from VentilationPattern

Once the termination criterion of an initiated inflation-type has been satisfied VentilationMode resumes its control function in accordance with the sequencing instructions from VentilationPattern

Possible inflation-types include:

| Abbreviated name/code | Full name | Inflation-type purpose-class applicability |
| --- | --- | --- |
| VC | volume-control | Assured |
| PC | pressure-control | Assured |
| PC(q) | flow-terminated pressure-control | Assured and Support |
| PS | pressure-support | Support |
| ES | effort-support | Support |
| ES[q+v] | effort-support based on changes in lung volume and airway flow | Support |
| vtPC | volume-targeted pressure-control | Assured |
| vtPS | volume-targeted pressure-support | Assured |
| PCVC | pressure-control switching unidirectionally to volume-control on condition | Assured |
| VCPC | volume-control switching bidirectionally to pressure-control on condition | Assured |

Inflation-type settings can include:

1	Inspiratory pressure

2	Δ Inspiratory pressure

3	Inspiratory flow

4	Target volume

5	Rise time

6	Termination criterion (e.g., time, flow, pressure, volume, EMG, set % peak flow)

7	High-pressure termination limit

8	Bias flow

9	Support/airway flow and/or Support/airway pressure

Notes to settings:

The term inspiratory pressure can be replaced by support pressure for the setting of pressure-support inflation-types.

Whenever an alternative termination criterion is used, there is always a set or preset time limit.

Inspiratory pressures are typically referenced to ambient pressure. Wherever they are referenced to the set BAP (PEEP) level, ISO 19223 requires their designation to be qualified with the preceding symbol Δ

The matrix below indicates which parameters, as settings, are associated with which inflation-type.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VC | N | N | Y | N | Y | Time | Y | Y | N |
| PC | Y | N | N | N | Y | Time | Y | Y | N |
| PC(q) | Y | N | N | N | Y | Flow | Y | Y | N |
| PS | N | Y | N | Y | Y | Flow | Y | Y | N |
| vtPC | N | N | N | Y | Y | Time | Y | Y | N |
| vtPS | N | Y | N | Y | Y | Flow | Y | Y | N |
| ES | ES is implemented by vendor-specific settings |  |  |  |  | Flow | Y | Y | N |
| ES[q+v] | N | N | N | N | N | Flow | Y | Y | Y |
| PCVC | Parameters that may be applicable to both PC and VC |  |  |  |  |  |  |  |  |
| VCPC | Parameters that may be applicable to both VC and PC |  |  |  |  |  |  |  |  |

#### AssuredInflationType (Class)

An assured inflation-type is one that has been selected for the delivery of assured inflations

This term is a purpose-classification, as distinct from the name of an inflation-type, which is a property-classification

Although assured to be delivered at the set rate, the assured inflation-type can also be initiated by a patient-trigger event in Group 1a ventilation-modes such as assist/control ventilation. Any such initiations can lead to deliveries of the assured inflation-type at a rate greater than that assured by the set rate. It follows that although all assured inflations are of the assured inflations-type, not all deliveries of the selected assured inflation-type occur because they are assured

An assured inflation-type can be initiated either by the ventilator or by a patient-trigger event.

#### AssuredInflationTypeSetting (Class)

Set quantities for assured inflation-types are the same as for inflation-types.

| Attribute | Notes |
| --- | --- |
| inflationTypeSetting CodeableConcept |  |
| setValue Quantity |  |

#### SupportInflationType (Class)

An inflation-type that has been selected to support a patient’s spontaneous breaths. This dictates that it is of a type that is intended to be terminated in response to the patient's respiratory activity and that it can only be used in ventilation-modes where its initiated is only ever in response to a patient-trigger event

This term is a purpose-classification, as distinct from the name of an inflation-type, which is a property-classification

The inflation-types typically selected as support inflation-types are pressure-support and effort-support inflation-types.

With support inflations, there is no assurance of delivery; they are so named because they are specified and configured to be used only to provide assistance to spontaneous breaths.

Support inflations can be selected to support spontaneous breaths between assured inflations, concurrent with assured inflations, or throughout complete assured-inflation cycles.

The breath associated with a support inflation is ‘supported’.

An effort-support inflation-type generates an airway-pressure waveform that is intended to be proportional to a patient's inspiratory effort. The required airway-pressure waveform is typically calculated moment-by-moment by a ventilator algorithm, using continuous measurements of parameters such as airway pressure, inspiratory flow, lung elastance, neural signals or EMG; it is generated by means of a pressure-regulation function tracking the calculated waveform. The intention of this inflation-type is to unload a proportion of the work of breathing during an inflation phase.

If the effort-support (ES) inflation is based on measurement of inspiratory flow, the support is most commonly determined as independent proportions of the resistive or elastance components of the inspiratory effort.

#### SupportInflationTypeSetting (Class)

Set quantities specific to SupportInflationTypes may include:

- Support pressure

- Δ Support pressure

- Effort-support criteria

Notes to settings:

The term support pressure has been included as a special case of an inspiratory pressure that may be used to denote the airway pressure during pressure-support inflations, primarily because its setting is often used alongside a setting for a pressure-control inflation on a user interface and in this way can be easily separately identified

The inspiratory pressure/support pressure of SupportInflationTypes is typically referenced to the set BAP(PEEP) level. ISO 19223 requires such designations to be qualified with the preceding symbol Δ

Its definition requires the termination criterion of a SupportInflationType to be responsive to the patient’s respiratory activity. For pressure-support inflations, this will, typically, be a set proportion of the peak inspiratory flow

| Attribute | Notes |
| --- | --- |
| inflationTypeSetting CodeableConcept |  |
| setValue Quantity |  |

#### 2nd SupportInflationType (Class)

For Group 2 ventilation-modes with an ACAP adjunct, a single pressure-support inflation setting can be arranged to support breaths throughout the full assured-inflation cycle if its set inspiratory-pressure is higher than the set inspiratory pressure for the assured inflations. Alternatively, a second support inflation-type can be selected for the support of any spontaneous breaths occurring during the assured inflation phases (concurrent breaths).

A selected second support inflation-type is typically of the pressure-support inflation-type. Its set support pressure is typically referenced to the set inspiratory pressure of the assured inflation-type and is designated as the set Δ H support pressure.

#### 2ndSupportInflationTypeSetting (Class)

Set quantities for 2nd support inflation-types are the same as for support inflation-types.

| Attribute | Notes |
| --- | --- |
| inflationTypeSetting CodeableConcept |  |
| setValue Quantity |  |

### Outline of respiratory support system

Figure 17 - Schematic view of respiratory support system

### Gases (Class)

Fresh gases are usually air, oxygen or nitrous oxide. The flows of fresh gas to the patient are regulated by the anesthetic machine/ventilator

### AnesthesiaWorkstation (Class)

Over the years, the conventional anesthesia machine has evolved into an advanced care station. The new machines use advanced electronics, software and technology to offer extensive capabilities for ventilation, monitoring, inhaled agent delivery, low-flow anesthesia and closed-loop anesthesia. They offer integrated monitoring and recording facilities and seamless integration with anesthesia information systems. It is possible to deliver tidal volumes accurately and eliminate several hazards associated with the low-pressure system and oxygen flush. Appropriate use can result in enhanced safety and ergonomy of anesthetic delivery and monitoring. (Divatia, J., Patil, V. and Shetmahajan, M., 2013. The modern integrated anaesthesia workstation. Indian Journal of Anaesthesia, 57(5), pp.446 - 454)

### VolatileAgentDeliveryDevice (Class)

This device mixes the required amount of a volatile anesthetic agent with the fresh gases. Most modern machines use positive pressure vaporizers that can deliver a reliable and accurately calibrated concentation to the patient

### Monitor (Class)

The anesthetic machine can provide a wide range of parameters relevant to respiratory status. These include:

- Respiratory Rate

- Tidal volume

- Fractional Inspired Oxygen Concentration

- End-tidal CO2 (EtCO2)

- Inspiratory Flow Time

- Expiratory time

- I:E Ratio (Inspired Time to Expired Time)

- Peak Airway Pressure

- Peak Inspiratory Pressure

- Continuous Positive Airway Pressure (CPAP)

- Positive End Expiratory Pressure

Divatia, J., Patil, V. and Shetmahajan, M., 2013. The modern integrated anaesthesia workstation. Indian Journal of Anaesthesia, 57(5), pp.446 - 454

### Ventilator (Class)

The anesthetic ventilator may be a separate component but is usually incorporated in the anesthesia machine along with some monitoring functions and one or more vaporizers. The ventilation patterns usually fewer than might be found in an ITU ventilator. The modeling of ventilation patterns can become very complex and is out of scope of this DAM.  Typically, only one pattern will be used for the entire procedure but occasionally the anesthesiologist might switch to another during the procedure.

The most common modes on anesthesia workstations, below includes both ventilation pattern and inflation type(s).  Before the dash is the pattern, after the dash is one or more inflation types separated by /. Note for all and especially CSV-PS the inspiratory pressure for the PS inflation type can equal zero. All of the patterns can also offer ACAP (Automatic Positive Airway Pressure) as an adjunct.

CMV-VC (Continuous Mandatory Ventilation-Volume Controlled)

CMV-PC  (Continuous Mandatory Ventilation-Pressure Controlled)

CMV-vtPC ( (Continuous Mandatory Ventilation-Target Pressure Controlled)

AC-VC (Assist Control - Volume Control)

AC-PC (Assist Control - Pressure Control)

AC-vtPC (Assist Control - Volume Target Pressure Control)

SIMV-VC (Synchronized Intermittent Mandatory Ventilation - Volume Control)

SIMV-PC (Synchronized Intermittent Mandatory Ventilation - Pressure Control)

SIMV-vtPC(Synchronized Intermittent Mandatory Ventilation - Volume Target Pressure Control)

SIMV-VC/PS (Pressure Support)

SIMV-PC/PS (Pressure Support)

SIMV-vtPC/PS (Pressure support)

CSV-PS (Continuous Spontaneous Ventilation (Pressure Support)

### Breathing System (Class)

Breathing systems link the anesthetic machine/ventilator to the airway. They may be open, semi-open or closed and rebreathing (using a CO2 absorber) or non-rebreathing. Many distinct types exist but they are generically classified using the Mapleson scheme which defines 6 basic types, labelled a - f. Breathing systems and their components are also known by name e.g. Lack, Bain, Ayre's T-piece.

| Attribute | Notes |
| --- | --- |
| categoryMapleson CodeableConcept | Mapleson A Mapleson B Mapleson C Mapleson D Mapleson E Mapleson F Semi-closed circle Closed circle |
| description CodeableConcept | Magill Lack Waters Bain Ayre's T-piece Ree's T-Piece |

### Airway (Class)

Airway devices provide a secure, patent, means to ventilate the unconscious patient. The most commonly used is the oral endotracheal tube but many other types are available (see Vocabulary Examples).

### Patient ConnectionPort (Class)

A connection between the breathing system and the airway that can provide a T-piece to support 'bag and mask' manual ventilation

### BagAndMask (Class)

A bag and mask allows manual ventilation of the patient

### Procedure

This pattern is modeled on the FHIR procedure resource which provides all the necessary attributes to document the anesthetic procedure and sub-procedures.

#### Procedure (Class)

An action that is or was performed on or for a patient. During intra-operative anesthesia, the main procedure (anesthesia episode) may comprise several major component procedures (e.g. induction, intubation, neuraxial block) which themselves may comprise identifiable sub-components. These relationships are captured by the 'part of' attribute that references the immediate parent of a procedure (if any).

| Attribute | Notes |
| --- | --- |
| identifier Identifier | A business identifier that allows this procedure instance to be referenced. This is particularly useful when procedures are nested i.e. a main procedure has one or more sub-procedures. An example would be an anesthesia episode that included an epidural anesthesia which, itself, included a procedure to insert an epidural catheter |
| instantiatesUri uri [0..*] | A URL pointing to an externally maintained protocol, guideline, order set or other definition that is adhered to in whole or in part by this Procedure. For example, an anesthesia department guideline or anesthetic plan for this case |

| Attribute | Notes |
| --- | --- |
| basedOn Reference | A reference to a resource that contains details of the request for this procedure. |
| part of Reference [0..*] | A larger event of which this particular procedure is a component or step. For example, a central venous line placement is a part of an anesthetic episode which, itself, is part of the overall surgical case |
| status code | A code specifying the state of the procedure. Generally, this will be the in-progress or completed state. |
| statusReason CodeableConcept [0..1] | The reason for the current state of the procedure. |
| category CodeableConcept [0..1] | A code that classifies the procedure for searching, sorting and display purposes (e.g. "Anesthetic Procedure"). |
| code CodeableConcept [0..1] | The specific procedure that is performed, for example: 'general anesthetic' SCTID_50697003 |
| subject Reference | The person on whom the procedure was performed |
| encounter Reference | The Encounter during which this Procedure was created or performed or to which the creation of this record is tightly associated. This will typically be the encounter the event occurred within, |
| recorder Reference [0..1] | Individual who recorded the record and takes responsibility for its content. |
| asserter Reference [0..1] | Individual who is making the procedure statement. |
| location Reference [0..1] | The location where the procedure actually happened, for example, 'operating room' |
| reasonCode CodeableConcept [0..*] | The coded reason why the procedure was performed. |
| reasonReference Reference [0..*] | The justification of why the procedure was performed. It is possible for a procedure to be a reason (such as C-Section) for another procedure (such as an epidural |
| bodySite CodeableConcept [0..*] | Detailed and structured anatomical location information. Multiple locations are allowed |
| outcome CodeableConcept [0..1] | The outcome of the procedure - did it resolve the reasons for the procedure being performed |
| note Annotation [0..*] | A free-text note |
| used Reference [0..*] | Identifies medications, devices and any other substance used as part of the procedure. |
| usedCode CodeableConcept [0..*] | Identifies coded items that were used as part of the procedure |

#### Performed (Class)

When the procedure was performed. Estimated or actual date, date-time, period, or age when the procedure was performed. Allows a period to support complex procedures that span more than one date, and also allows for the length of the procedure to be captured.

| Attribute | Notes |
| --- | --- |
| dateTime dateTime | The time when the procedure was performed |
| period Period | The period of the procedure, defined by start and end times |
| phase CodeableConcept | A phase of the main anesthetic encounter when the procedure was performed e.g. induction, emergence |

#### Performer (Class)

The person who performed the procedure

| Attribute | Notes |
| --- | --- |
| performer CodeableConcept | The role of the performer in the procedure |
| actor Reference | The reference to the practitioner |
| onBehalfOf Reference | Organization the device or practitioner was acting for |

#### Complication (Class)

Any complications that occurred during the procedure, or in the immediate post-performance period

| Attribute | Notes |
| --- | --- |
| complication CodeableConcept | The complication |
| complicationStatus CodeableConcept | A code that indicates the seriousness / reportability of the complication for reporting, clinical governance and quality assurance processes |

#### FollowUp (Class)

Any required follow-up

| Attribute | Notes |
| --- | --- |
| followUp CodeableConcept | The particular followup required |
| list CodeableConcept | A list to which this follow up should be added e.g. post-operative instructions |

#### Note (Class)

Any other notes and comments about the procedure.

| Attribute | Notes |
| --- | --- |
| note Annotatiom | Free text annotation providing more information about the procedure |

### Practitioner and Care Team

A person with a formal responsibility for providing healthcare or related services. A practitioner may be a member of a Care Team and may possess one or more professional qualifications or none.

During a procedure the practitioner may perform one or more procedural roles and separately. may also supervise or be supervised by, another practitioner.

In all cases, the period(s) during which a role and/or supervision applied is important to know.

Figure 18 - Practitioner & Care Team

#### CareTeam (Class)

The Care Team includes all the people and organizations who plan to participate in the coordination and delivery of care for a patient.

| Attribute | Notes |
| --- | --- |
| category codeableConcept | Identifies what kind of team. This is to support differentiation between multiple co-existing teams, such as care plan team, episode of care team, longitudinal care team. E.g. LA27976-2 (Encounter-focused care team) |
| encounter Reference (Encounter) | The Encounter during which this CareTeam was created or to which the creation of this record is tightly associated. |
| identifier InstanceIdentifier | Business identifier(s) assigned to this care team by the performer or other systems which remain constant as the resource is updated and propagates from server to server. |
| managingOrganization Reference(Organization) | The organization responsible for the care team. |
| name string | A label for human use intended to distinguish like teams. E.g. the "red" vs. "green" trauma teams. |
| note Annotation | Comments made about the CareTeam. |
| period Period | Indicates when the team did (or is intended to) come into effect and end. |
| reasonCode CodeableConcept | Describes why the care team exists. |
| reasonReference Reference(Condition) | Condition(s) that this care team addresses. |
| status CodedValue | Indicates the current state of the care team. Values are: proposed, active, suspended, inactive, enter-in-error |
| subject Reference(Patient) | Identifies the patient or group whose intended care is handled by the team. |
| telecom ContactPoint | A central contact detail for the care team (that applies to all members). |

#### participant (Class)

Identifies all people and organizations who are expected to be involved in the care team.

| Attribute | Notes |
| --- | --- |
| member Reference(Practitioner) | The specific person or organization who is participating/expected to participate in the care team. |
| onBehalfOf Reference(Organization) | The organization of the practitioner. |
| period period | Indicates when the specific member or organization did (or is intended to) come into effect and end. |
| role CodeableConcept | Indicates specific responsibility of an individual within the care team, such as "Primary care physician", "Trained social worker counselor", "Caregiver", etc. See SCTID 223366009 (Healthcare professional) |

#### Practitioner (Class)

A person with a formal responsibility for providing healthcare or related services.

| Attribute | Notes |
| --- | --- |
| identifier Identifier | A unique identifier that allows this practitioner to be identified in the system. This can be used to associate a practitioner with an act. |
| firstName string | The first name of the practitioner. The entire name can be represented using the HL7 datatype 'human name' but first name/last name are used in this class for the purposes of clarity in the model. |
| lastName string | The first name of the practitioner |
| active boolean | Whether this practitioner's record is in active use. |
| gender CodeableConcept | Administrative Gender - the gender that the person is considered to have for administration and record keeping purposes.. |

#### SupervisionRole (Class)

The role(s) if any played by the practitioner in relation to supervision. Examples:

- Supervisor

- Supervisee

| Attribute | Notes |
| --- | --- |
| supervisor_id id | The identity of the supervisor |
| supervisee_id id | The identity of the supervisee |
| supervisionType CodeableConcept | The nature of the supervision. Examples: Direct physical supervision Periodic supervision Remote supervision |
| period period | The period during which this supervision was provided |

#### ProceduralRole (Class)

The role(s) played by the practitioner related to the procedure

| Attribute | Notes |
| --- | --- |
| description string | The role of the practitioner in relation to a procedure |
| code CodeableConcept | A code representing the role |
| period Period | The period during which the role was active |

#### Speciality (Class)

A particular specialty or skill possessed by the practitioner. This is not the same as a qualification. For example, an anesthesiologist, qualified to provide a range of anesthesia services, may have specialized experience of pediatric anesthesia

| Attribute | Notes |
| --- | --- |
| speciality string | Description of specialization e.g. neuro-anesthesthesiologist, cardiac anesthesiologist. |
| experience annotation | A free text description of special training or experience relevant to the competence of the practitioner to carry out particular procedures or tasks or to particular classes of of patient or case e.g. children, adults, neurosurgical. |

The official certifications, training, and licenses that authorize or otherwise pertain to the provision of care by the practitioner. For example, a medical license issued by a medical board authorizing the practitioner to practice medicine within a certain jurisdiction.

| Attribute | Notes |
| --- | --- |
| identifier string | An identifier for this qualification for the practitioner. |
| code CodeableConcept | Coded representation of the qualification. |
| description string | A plain language description of the qualification. Examples: MD CRNA FRCA |
| period period | Period during which the qualification is valid. Qualifications may only be for a limited period of time, and can be revoked. |
| issuer ref | Reference (Organization) Organization that regulates and issues the qualification. |

#### ProfessionalRegistration (Class)

Professional bodies/jurisdictions maintain registers of individuals licensed to practice medicine.  Examples in the US are: National Provider Number, State Licensing Number and Drug Enforcement Agency (DEA) Number. In the UK a national register is maintained by the General Medical Council (GMC).

| Attribute | Notes |
| --- | --- |
| identifier string | An identifier for this qualification for the practitioner. Examples: US: National Provider Number State Licensing Numbers Drug Enforcement Numbers |
| code CodeableConcept | Coded representation of the registration |
| description string | A plain language description of the registration |
| period period | Period during which the registration is valid. Registrations may only be for a limited period of time, and can be revoked. |
| issuer ref | Reference (Organization) Organization that regulates and issues the registration. |

### Checks & Safety

Figure 19 - Checks & Safety

#### WHO Surgical Safety Checklist (Class)

The WHO Surgical Safety Checklist was developed after extensive consultation aiming to decrease errors and adverse events and increase teamwork and communication in surgery. The 19-item checklist has gone on to show significant reduction in both morbidity and mortality and is now used by a majority of surgical providers around the world.

https://www.who.int/patientsafety/safesurgery/checklist/en/

| Attribute | Notes |
| --- | --- |
| checklistCompleted boolean | A simple confirmation, true if the checklist is done |
| doneBy Reference (Practioners) | A reference to the practitioner who performed the checks |
| verifiedBy Reference (Practioner) | A reference to the practioner who verified the checks |

#### SignIn (Class)

Preliminary phase during which the patient's identify is confirmed and some risks are reviewed

#### Patient Confirmation (Class)

Before surgery it is required to confirm the patient's identity

| Attribute | Notes |
| --- | --- |
| patientID boolean | True if the patient has confirmed their identity |
| surgicalSite boolean | True if the patient confirms that the intended surgical site is correct in their understanding |
| procedure boolean | True if the patient confirms that the intended procedure is correct in their understanding |
| consent boolean | True if the patient (or their representatiive) has given consent for the procedure |

#### Aspiration Risk (Class)

This records if there is estimated to be an aspiration risk and what precautions are taken to mitigate such risk. Aspiration risk is likely to have been identified in the Pre-assessment and to have influenced the anesthetic plan

| Attribute | Notes |
| --- | --- |
| aspirationRisk boolean | Tru if an aspiration risk has been identified |
| equipmentAvailable boolean | True if equipment is available in the case of aspiration e.g. suction apparatus |
| assistanceAvailable boolean | True if assistance is available should it be needed |

#### Blood Loss Risk (Class)

This records if there is estimated to be a risk of blood loss and what precautions are taken to mitigate such risk. Blood loss risk is likely to have been identified in the Pre-assessment and to have influenced the anesthetic plan

| Attribute | Notes |
| --- | --- |
| bloodLossRisk boolean |  |
| IVaccessFluids boolean | True if both intravenous access and appropriate fluids are available |

#### Time Out (Class)

A time-out, is a deliberate pause by the entire care team to confirm, amongst other things, the correct patient, procedure, and site

| Attribute | Notes |
| --- | --- |
| confirmTeamRoles boolean | Asserts that all team members have introduced themselves by name and role |
| antibioticProphylaxis boolean | True if antibiotic phriphylaxis has been instated within the last 60 minutes |
| imagingDisplayed boolean | True if essential imaging is displayed |

#### PatientProcedureSite (Class)

A confirmation by the whole team that the patient, procedure site and the procedure are agreed to be correct and as intended

| Attribute | Notes |
| --- | --- |
| patient boolean | True of the patient identity is confirmed |
| site boolean | True if the procedural site is confirmed |
| procedure boolean | True if the procedure is confirmed |

#### AnticipatedEvent (Class)

Anticipated Critical Events

Surgeon reviews: what are the critical or unexpected steps, operative duration, anticipated blood loss?

Anesthesia team reviews: are there any patient-specific concerns?

Nursing team reviews: has sterility (including indicator results) been confirmed? Are there equipment issues or any concerns?

#### Event (Class)

An event that is anticipated. During the surgical episode it is necessary to record a number of events which can be broadly classified as:

- Time out - a period of time designated for a review of some aspect of the case by the care team. An example is to check that the surgical site is marked, that the laterality is correct, and that the patient has given consent to the planned procedure

- Milestone events - significant events that mark the start, progress and finish of the procedure. These include the start and end of surgery and anesthesia, induction, reversal, transfer of the patient between locations etc.

- Procedural events - a note of when something was done like 'intubation', 'induction', 'diathermy'

- Adverse events - events that cause harm, or have the potential to cause harm, to the patient and that are not the caused by the underlying condition

These categories allow events to be grouped for the purposes of documentation. Some events can be anticipated, and this should be recorded as part of the ‘timeout’ before the procedure is started.

| Attribute | Notes |
| --- | --- |
| eventCategory CodeableConcept [0..1] | The category of event for example: Timeout, Milestone, Procedural, Adverse |
| eventName CodeableConcept | The specific name of the event e.g. 'difficult intubation', 'bronchospasm', 'airway device cuff herniation', 'induction of general anesthesia' |
| eventTime dateTime | The time when the event is asserted to have occurred ('Observation' provides a more sophisticated resource for recording more details concerning the authorship and timing of events) |
| eventNote Annotation | A free text annotation of the event |

#### ASA Checklist (Class)

This is based on the 'Guidelines for Pre-Anesthesia Checkout Procedures' published by the ASA.  There are 15 items and the checklist requires that the responsible person documents that each has been checked and confirmed.

The 'checkoutComplete' attribute is a top-level indication that all the checks have been done.

| Attribute | Notes |
| --- | --- |
| checklistCompleted boolean | This indicates the overall status of the checklist i.e. all items either have or have not, been checked as OK. In many cases, in the anesthetic record this summary status indication is all that will be recorded. |
| doneBy Reference (Practioners) | Who did the checks |
| verifiedBy Reference (Practioner) | Who verified the checks |
| acPower boolean | Turn on anesthesia delivery system and confirm that ac power is available. |
| manualVentilation boolean | Verify Auxiliary Oxygen Cylinder and Self-inflating Manual Ventilation Device are Available & Functioning |
| monitoringAlarms boolean | Verify availability of required monitors, including alarms. |
| patientSuction boolean | Verify patient suction is adequate to clear the airway |
| spareO2Pressure boolean | Verify that pressure is adequate on the spare oxygen cylinder mounted on the anesthesia machine |
| pipedGasPressure boolean | Verify that the piped gas pressures are = 50 psig |
| vaporizers boolean | Verify that vaporizers are adequately filled and, if applicable, that the filler ports are tightly closed. |
| lowPressureSystem boolean | Verify that there are no leaks in the gas supply lines between the flowmeters and the common gas outlet |
| scavenging boolean | Test scavenging system function |
| oxygenMonitor boolean | Calibrate, or verify calibration of, the oxygen monitor and check the low oxygen alarm. |
| CO2Absorbent boolean | Verify carbon dioxide absorbent is not exhausted |
| breathingSystem boolean | Breathing system pressure and leak testing. |
| gasFlowsInCircuit boolean | Verify that gas flows properly through the breathing circuit during both inspiration and exhalation |
| ventilatorSettings boolean | Confirm ventilator settings and evaluate readiness to deliver anesthesia care |

#### PatientStatus (Class)

A list of basic items relevant to the patient's preparedness.

| Attribute | Notes |
| --- | --- |
| lastFood dateTime | The time when the patient last consumed solid food |
| lastOralFluid dateTime | The time when the patient last had a drink |
| emotion enum | The patient's emotional state: calm anxious crying |
| consciousness enum | Detailed appraisal of consciousness may be done using, for example, the Glasgow Coma Scale. However, in the case of this check it is sufficient to record whether the patient was Awake Drowsy Asleep |

#### Patient Protection (Class)

Summary of patient protection measures. These may be recorded in more detail in the context of specific procedures such as patient positioning

| Attribute | Notes |
| --- | --- |
| eyesLubricated boolean | Eyes are lubricated |
| eyeShield boolean | Eyes are protected by a purpose designed shield |
| eyesPadded boolean | Eyes are protected by a gauze pad |
| eyesTaped boolean | Eyelids are held closed by tape |
| warmingBlanket boolean | A warming blanket is used to maintain homeostasis |
| biteGuard boolean | A bite guard is used to mitigate risk of injury during intubation |

#### PatientProtectionSite (Class)

Patient's require protection against a variety of hazards during surgical / anesthetic procedures. These include hypothermia, injury as a result of a change in position or of a position remaining unchanged for too long, drying of the eyes, damage to dentition etc. Teeth are at particularly at risk of being damaged by airway maintenance procedures, peripheral nerves and pressure points may require padding, the position of the patient, spine and limbs may need to be stabilized/secured, and so on.

| Attribute | Notes |
| --- | --- |
| anatomicalStucture CodeableConcept | The structure, eye, nerve, teeth to be protected |

#### Protection Method (Class)

Patient's require protection against a variety of hazards during surgical / anesthetic procedures. These include hypothermia, injury as a result of a change in position or of a position remaining unchanged for too long, drying of the eyes, damage to dentition etc. Teeth are at particularly at risk of being damaged by airway maintenance procedures, peripheral nerves and pressure points may require padding, the position of the patient, spine and limbs may need to be stabilized/secured, and so on.

| Attribute | Notes |
| --- | --- |
| period Period | The period of time that the protection method was employed |
| protectionMethod CodeableConcept | The specific method used to protect an anatomical structure. For example: Eyes taping the eyes closed ointment occlusive dressing bio-occlusive dressing goggles suture |

#### Dentition (Class)

Dental damage is the leading cause of complaint and medicolegal claims against anesthesiologists.

Direct laryngoscopy is implicated in most cases of dental injury.

Severe periodontitis is an independent risk factor for cardiovascular disease.

A thorough preoperative dental assessment, explanation of risk, and clear documentation in the

anesthetic record is essential, particularly in individuals with risk factors for dental injury.

Departments should have an agreed management pathway for patients who have sustained dental injury as prompt treatment can improve tooth survival.

Abeysundara, L., Creedon, A. and Soltanifar, D. (2016). Dental knowledge for anaesthetists. BJA Education, 16(11), pp.362-368.

This check supports the recording of dental status prior to the procedure, and also any damage caused by the procedure

| Attribute | Notes |
| --- | --- |
| looseTooth boolean [0..1] | Records that a loose tooth, or teeth, is noted |
| crown boolean [0..1] | Records the presence of one nor more crowns |
| bridge boolean | Records the presence of a bridge |
| periodontalDisease boolean [0..1] | Records the presence of periodontal disease |

#### Dentition Detail (Class)

Provides more specific information concerning dentition

| Attribute | Notes |
| --- | --- |
| finding CodeableConcept | The specific issue e.g. broken tooth, loose tooth, loose crown |
| locationSystem CodeableConcept | The system for identifying individual teeth. Several methods for the identification of individual teeth exist. In the USA, the Universal Numbering System is primarily used. Adult teeth are labelled with a number 1–32 starting with the third right maxillary molar and moving in an anti-clockwise direction. For primary dentition, letters A–T are used. The Palmer notation is most commonly used by dentists in the UK. Symbols (+ ++ +) mark the relevant quadrant and teeth are numbered 1–8 starting in the midline, as viewed by the assessor. For primary dentition, the letters A–E replace the numbers. The Palmer notation is further adapted in the ‘letters and numbers’ system; each quadrant is assigned two letters ( for instance, LL, left lower; RU, right upper) followed by a number 1–8. Internationally, the World Health Organization’s ISO system is often used. Each quadrant is designated a number 1–4, and each individual tooth a further number 1–8 as per the Palmer notation. |
| location string | The specific location using the notation from the relevant 'locationSystem' |

### Observation

Figure 20 - Observation

#### Observation (Class)

Measurements and simple assertions made about a patient, device or other subject. Examples in the anesthesia domain are:

- Observations relating to guidance during the insertion of a line including, verification of the final position.

- Observations concerning minor events such as an encounter with an obstruction during insertion of an epidural needle

- A procedural milestone

- A routine procedural event

- An adverse incident

| Attribute | Notes |
| --- | --- |
| identifier Identifier [0..*] | A unique identifier assigned to this observation. This is a business identifier and there may be more than one for any given observation |
| basedOn ref [0..*] | A plan, proposal or order that is fulfilled in whole or in part by this event. For example, a clotting time may need to be ascertained before a procedure can be started. |
| partOf Reference [0..*] | A larger event of which this particular Observation is a component or step. For example, an observation as part of a procedure. |
| status code | Observation.status: registered, preliminary, final, amended, corrected, cancelled, entered in error, unknown |
| category CodeableConcept [0..*] | A code that classifies the general type of observation being made. In addition to the required category valueset, this element allows various categorization schemes based on the owner’s definition of the category and effectively multiple categories can be used at once. The level of granularity is defined by the category concepts in the value set. In the context of this DAM the value sets are mainly taken from the SNOMED CT Anesthesia Terms Set that is maintained by the SNOMED CT Anesthesia Clinical Reference Group (CRG) |
| code CodeableConcept | Describes what was observed aka the observation "name". |
| subject Reference [0..1] | The patient, or group of patients, location, or device this observation is about and into whose record the observation is placed. If the actual focus of the observation is different from the subject (or a sample of, part, or region of the subject), the focus element or the code itself specifies the actual focus of the observation. In the context of the intra-procedural anesthesia episode all procedures and observations can inherit the identity of the anesthetized patient |
| focus Reference [0..*] | The actual focus of an observation when it is not the patient of record.This could be an event, an intervention, another observation of the subject, a body structure or inserted device. |
| encounter Reference [0..1] | The healthcare event (e.g. a patient and healthcare provider interaction) during which this observation is made i.e. the intra-procedural anesthesia episode |
| effective X | The time or time-period the observed value is asserted as being true. For biological subjects - e.g. human patients - this is usually called the "physiologically relevant time". This is usually either the time of the procedure or of specimen collection, but very often the source of the date/time is not known, only the date/time itself. The X data type indicated that one of a number of data types might be used |
| issued dateTime [0..1] | The date and time this version of the observation was made available to providers, typically after the results have been reviewed and verified. |
| performer Reference [0..*] | Who is responsible for the observation |
| value qty [0..1] | The information determined as a result of making the observation, if the information has a simple value. An observation may have; 1) a single value here, 2) both a value and a set of related or component values, or 3) only a set of related or component values. If a value is present, the datatype for this element should be determined by Observation.code. A CodeableConcept with just a text would be used instead of a string if the field was usually coded, or if the type associated with the Observation.code defines a coded value. Datatypes are: Quantity, CodeableConcept, string, boolean, integer, Range, Ratio, SampledData, time, dateTime, Period |
| dataAbsentReason CodeableConcept [0..1] | Provides a reason why the expected value in the element Observation.value[x] is missing. |
| interpretation CodeableConcept | A categorical assessment of an observation value. For example, high, low, normal. For the purposes of this DAM it could also be a term such as 'accidental dural puncture |
| note Annotation [0..*] | Comments about the observation or the results. May include general statements about the observation, or statements about significant, unexpected or unreliable results values, or information about its source when relevant to its interpretation. |
| bodySite CodeableConcept | Observed body part - indicates the site on the subject's body where the observation was made (i.e. the target site). Only used if not implicit in code found in Observation.code |
| bodySiteStatus CodeableConcept | A body site is not always known with certainty. For example, fluid aspirated from an epidural needle may be thought to come from the epidural space. However, it is possible, in the event of an inadvertent dural puncture, that it is from the sub-arachnoid space. This might be established by measuring, for example, the pH/temperature/glucose of an aspirate. In the event that the observation shows that the aspirate is CSF, then the body site can be confirmed, or it might be confirmed independently by imaging. This attribute indicates whether the status of the 'bodySite' attribute is 'Provisional' or 'Confirmed' |
| method CodeableConcept | How it was done - indicates the mechanism used to perform the observation. So, CO2 might be measured by capnography, pH by a litmus strip |
| specimen Reference | Specimen used for this observation |
| device Reference [0..1] | The device used to generate the observation data. This could be an utrasound machine used to establish an anatomical structure or to confirm the correct positioning of a line |
| hasMember Reference [0..*] | This observation is a group observation (e.g. a battery, a panel of tests, a set of vital sign measurements) that includes the target as a member of the group. |
| derivedFrom Reference | The target resource that represents a measurement from which this observation value is derived. For example, a calculated anion gap or a fetal measurement based on an ultrasound image. |

#### Identifier (Class)

A unique identifier assigned to this observation. This is a business identifier and there may be more than one for any given observation

| Attribute | Notes |
| --- | --- |
| identifier identifier |  |

#### Based On (Class)

A plan, proposal or order that is fulfilled in whole or in part by this event. For example, a clotting time may need to be ascertained before a procedure can be started.

| Attribute | Notes |
| --- | --- |
| basedOn ref |  |

#### partOf (Class)

A larger event of which this particular Observation is a component or step. For example, an observation as part of a procedure.

| Attribute | Notes |
| --- | --- |
| partOf ref | A larger event of which this particular Observation is a component or step. For example, an observation as part of a procedure. |

#### Category (Class)

A code that classifies the general type of observation being made.

In addition to the required category value set, this element allows various categorization schemes based on the owner’s definition of the category and effectively multiple categories can be used at once. The level of granularity is defined by the category concepts in the value set.

In the context of this DAM the value sets are mainly taken from the SNOMED CT Anesthesia Terms Set that is maintained by the SNOMED CT Anesthesia Clinical Reference Group (CRG)

| Attribute | Notes |
| --- | --- |
| category CodeableConcept |  |

#### Focus (Class)

The actual focus of an observation when it is not the patient of record representing something or someone associated with the patient such as a spouse, parent, fetus, or donor.

The focus of an observation could also be an existing condition, an intervention, the subject's diet, another observation of the subject, or a body structure such as tumor or inserted device.

| Attribute | Notes |
| --- | --- |
| focus ref |  |

#### Performer (Class)

The person responsible for asserting the observed value as "true".

In the context of this DAM this will usually be the clinician of reference e.g. the anesthesiologist

| Attribute | Notes |
| --- | --- |
| performer ref |  |

#### Interpretation (Class)

A categorical assessment of an observation value. For example, high, low, normal. For the purposes of this DAM it could also be code such as 'accidental dural puncture'

| Attribute | Notes |
| --- | --- |
| interpretation CodeableConcept |  |

#### Note (Class)

Comments about the observation or the results.

May include general statements about the observation, or statements about significant, unexpected or unreliable results values, or information about its source when relevant to its interpretation.

| Attribute | Notes |
| --- | --- |
| note string |  |

#### Component (Class)

Some observations have multiple component observations. These component observations are expressed as separate code value pairs that share the same attributes. Examples include systolic and diastolic component observations for blood pressure measurement and peak amplitudes in a train-of-four measurement.

Component observations share the same attributes in the Observation resource as the primary observation and are always treated a part of a single observation (they are not separable). However, the reference range for the primary observation value is not inherited by the component values and is required when appropriate for each component observation.

| Attribute | Notes |
| --- | --- |
| code CodeableConcept | Describes what was observed. Sometimes this is called the observation "code". Requirements Knowing what kind of observation is being made is essential to understanding the observation. |
| value Quantity |  |
| valueCodeableConcept CodeableConcept |  |
| dataAbsentReason CodeableConcept |  |
| interpretation CodeableConcept |  |

#### SubstanceSample (Class)

A sample of a substance directly associated with an anesthetic procedure. Examples are: fluids aspirated from an invasive device such as an epidural needle, or a gas in a breathing circuit

| Attribute | Notes |
| --- | --- |
| sourceDevice Reference [0..1] | A device from which the sample is obtained e.g. an epidural needle, arterial line, breathing circuit |
| substanceType CodeableConcept | The type of substance: liquid, gas etc. |

### Findings and Events

The FHIR observation resource, on which the 'Observation' package, is based will handle all general observations. This package highlights types of finding of especial relevance to intra-operative anesthesia that reflect the requirements of relevant clinical notes.  In many cases findings during intra-operative anesthesia are relatively simple and can be recorded by using a term from the IOTA term set with the addition of an asserter (usually the anesthesiologist of record) and a timestamp. Other findings require slightly more detail to be recorded: common examples are included here.

#### Event (Class)

During the surgical episode it is necessary to record a number of events which can be broadly classified as:

- Time out - a period of time designated for a review of some aspect of the case by the care team. An example is to check that the surgical site is marked, that the laterality is correct, and that the patient has given consent to the planned procedure

- Milestone events - significant events that mark the start, progress and finish of the procedure. These include the start and end of surgery and anesthesia, induction, reversal, transfer of the patient between locations etc.

- Procedural events - a note of when something was done like 'intubation', 'induction', 'diathermy'

- Adverse events - events that cause harm, or have the potential to cause harm, to the patient and that are not the caused by the underlying condition

These categories allow events to be grouped for the purposes of documentation

| Attribute | Notes |
| --- | --- |
| eventCategory CodeableConcept [0..1] | The category of event for example: Timeout, Milestone, Procedural, Adverse |
| eventName CodeableConcept | The specific name of the event e.g. 'difficult intubation', 'bronchospasm', 'airway device cuff herniation', 'induction of general anesthesia' |
| eventTime dateTime | The time when the event is asserted to have occurred ('Observation' provides a more sophisticated resource for recording more details concerning the authorship and timing of events) |
| eventNote Annotation | A free text annotation of the event |

#### SubstanceFinding (Class)

Sometimes a sample is derived from an invasive device and its nature is determined by appropriate tests. A common example would be the aspiration of fluid from an epidural needle or catheter. The presence of blood indicates that a blood vessel has been penetrated.  A pH of around 7.33 and/or protein of around 0.2g/l may suggest that the sample is CSF. A temperature lower than around 37 degrees Celsius may suggest that the aspirate is saline (used in Loss of Resistance (LOR) technique) rather than CSF which should be at body temperature.

Another example is the use of capnography to establish that an ET tube is in the trachea and not the esophagus. In the former case, CO2 will be detected in the expired breath whereas its absence indicates incorrect placement.

Since the point of acquiring and testing the sample is often to confirm its nature and source, the source may be presumed in the first instance and then confirmed or otherwise after appropriate evaluation of the sample. It is therefore necessary (and useful) to record the presumed and confirmed source, separately.

| Attribute | Notes |
| --- | --- |
| sourceDevice Reference (Device) | The source of the substance in terms of the device from which it is acquired e.g. epidural catheter, central line, ET tube |
| presumedSource CodeableConcept | The presumed or expected source (anatomical structure) from which the sample has come |
| confirmedSource CodeableConcept | The confirmed source (anatomical structure) from which the sample has come, identified after evaluation of the sample |
| presumedNature CodeableConcept | The presumed nature of the sample e.g. CSF, venous blood |
| confirmedNature CodeableConcept | The confirmed nature of the sample e.g. saline, arterial blood |
| type CodeableConcept | Substance type e.g. gas, fluid |
| sampleDescription CodedConcept | A description of the sample |
| confirmedBy CodeableConcept | Method of confirmation |

#### Test (Class)

A test of a substance to determine its likely type and source e.g. glucose, protein, pH

| Attribute | Notes |
| --- | --- |
| method CodeableConcept [0..*] | The measurement method e.g. capnography, glucose test strip |
| measuredParameter CodeableConcept | The measured parameter e.g. glucose, CO2, temperature |
| parameterValue Quantity |  |
| finding CodeableConcept | The finding e.g. heme, cloudy appearance, warm, CO2 |
| interpretation CodeableConcept | Clinical interpretation of the finding. For example, if fluid aspirated from an epidural needle contained blood then accidental venous puncture might be suspected. If a sample of blood was bright red, an arterial source might be suspected. |

#### DevicePosition (Class)

The anatomical position of a device

| Attribute | Notes |
| --- | --- |
| id identifier | Unique instance identifier (business identifier) assigned to the device |
| deviceName CodeableConcept | The name of the device e.g. 'oral endotracheal tube' |
| bodySite CodeableConcept |  |
| confirmationTechnique CodeableConcept | Examples of confirmation technique are, ultrasound, substance finding, test dose, flouroscopy and X-ray |
| relativeToInsertionSite Quantity | The device position relative to the intended insertion site e.g. needle tip from skin, ET tube from incisors |
| relativeToTargetSite Quantity | The position of the device relative to the intended target site |
| freeLength Quantity | The length of the device beyond the first point of entry to the target structure for example, the length of catheter in the epidural space |

#### Visualization&Inspection (Class)

This refers to the straightforward visual or other simple identification of an anatomical target or device position. Non-visual methods include palpation and auscultation

| Attribute | Notes |
| --- | --- |
| method CodeableConcept | The method can be visual inspection, palpation or auscultaion |
| direct boolean | Direct visualization involves a line-of-sight to the target. This may require the displacement of structures that obscure a clear view. An example is the use of a basic laryngoscope to hold the tongue to one side so that the glottis can be seen. |
| indirect boolean | Indirect visualization normally involves optical devices such as fiberoptic bronchoscopes, video laryngoscopes, fiberoptic stylets and mirror or prism optically-enhanced laryngoscopes. |
| device Reference [Device] | A reference to a device used for indirect visualization of an anatomical structure, or the position of a device. Examples are: video laryngoscope, fiber-optic laryngoscope |

#### LocationByLandmark (Class)

Surface landmarks are commonly used to locate the point where a device should be inserted e.g. a central line or epidural catheter. In some cases, a single landmark e.g. an inter-vertebral space found by counting down the spine is sufficient. Other times a position needs to be established by reference to several surface landmarks. This derivation is often by drawing an imaginary line (or lines) between landmarks e.g. the intercristal line that joins the supra-iliac crests and crosses the spine at the L3/L4 level.  More complex derivations are also used.

| Attribute | Notes |
| --- | --- |
| targetSite CodeableConcept | The site that requires to be identified using surface landmarks |
| derivedMeasure CodeableConcept | A measure derived from the identified landmarks e.g. inter-cristal line |
| measurement boolean | True if identification of the required site has involved measurement from landmarks that have been identified and/or derived measures |
| note Annotation |  |

#### Landmark (Class)

A defined anatomical structure that serves as a reference point

| Attribute | Notes |
| --- | --- |
| bodySite CodeableConcept | The identity of the landmark |
| identificationMethod CodeableConcept [0..*] | The identification method may be simple inspection, palpation, counting etc. |

#### NerveStimulation (Class)

This describes the use of a peripheral nerve stimulator (PNS) to elicit a twitch response. The presence and size of the response can be used to assess the proximity of the stimulating device to a target nerve. The response to stimulation depends on a number of factors including the position of the electrodes but most importantly, the parameters of stimulation.

| Attribute | Notes |
| --- | --- |
| nerve CodeableConcept | The nerve that is stimulated e.g. ulnar nerve in TOF technique |
| twitchSite CodeableConcept | The bodysite where a response to stimulation is observed |
| numberOfTwitches int | The number of twitches observed using the established level for a supra-maximal current |
| TOFRatio Quantity | In Train-of-Four (ToF) a set of 4 stimuli is given at a rate of about two a second and the twitch response to each is measured. From these, the ToFR ratio (ToFR) is calculated by dividing the amplitude of the fourth response by the amplitude of the first response. |

#### Ultrasound (Class)

The use of an ultrasound machine to identify an anatomical location and / or the position  of a device.

| Attribute | Notes |
| --- | --- |
| inPlane boolean | With the in-plane approach, the device, say, a needle, enters the skin at the side of the probe. The needle traverses the plane of ultrasound and the whole shaft is visualized as it progresses towards the target. |
| outOfPlane boolean | With the out-of-plane approach, the device, say, a needle, enters the skin away from the probe, and is aimed at the plane of sound. With this approach just the needle tip is visualized, and the remainder of the needle is off screen |
| doppler boolean | Doppler ultrasound is a mode that is useful to highlight the position of blood vessels so that the operator can avoid penetrating a vessel while introducing a needle. |

#### SensoryBlock (Class)

The assessment of sensory block in a conscious patient generally involves the measurement of the dermatome level of the block, bilaterally, to a stimulus such as touching with a cold substance (ice), cold object (metal), application of ethyl chloride, pricking the skin with a sharp object (toothpick etc. The first step is to establish a reference level by applying the stimulus to a site that is definitely unaffected by the block and then comparing sensation to that reference as the stimulus is applied at different sites.

| Attribute | Notes |
| --- | --- |
| dermatome CodeableConcept | A dermatome is an area of sensory nerves close to the skin that are supplied by a specific spinal nerve root. |
| laterality CodeableConcept | The laterality of the measurement, left, right or bilateral |
| strimulusType CodeableConcept | Cold, pinprick, electrical stimulation |

#### MotorBlock (Class)

Gross motor block in conscious patients is usually assessed by asking the patient to perform movement of the lower limbs. The degree of block is scored using the Bromage Scale where: I = Free movement of legs and feet, II = Just able to flex knees with free movement of feet  III = Unable to flex knees, but with free movement of feet and, IV = unable to move legs or feet. A variant of the Bromage scale (modified Bromage) uses a similar assessment but with 6 levels instead of four.

| Attribute | Notes |
| --- | --- |
| bromageScore CodeableConcept | A score based on the degree of voluntary movement of the lower limbs |
| laterality CodeableConcept | The laterality of the measurement, left, right or bilateral |
| modifiedBromageScore CodeableConcept | Based on the Bromage score but with more levels |
| trainOfFour CodeableConcept | A technique for detecting the degree of motor blockade produced by muscle relaxants. A train of four electrical stimuli are administered and the muscle twitch response, noted. The number of twitches is a measure of the degree of blockade. A derived measure is the train of four ratio. |

### AllergyIntolerance

#### AllergyIntolerance (Class)

Risk of harmful or undesirable, physiological response which is unique to an individual and associated with exposure to a substance.

| Attribute | Notes |
| --- | --- |
| identifier identifier [0..*] | Allows identification of the AllergyIntolerance as it is known by various participating systems and in a way that remains consistent across servers. |
| clinicalStatus CodeableConcept [0..1] | The clinical status of the allergy or intolerance. |
| verificationStatus CodeableConcept [0..1] | Assertion about certainty associated with the propensity, or potential risk, of a reaction to the identified substance (including pharmaceutical product). |
| type code [0..1] | Identification of the underlying physiological mechanism for the reaction risk. |
| category code [0..*] | Category of the identified substance |
| criticality code [0..1] | Estimate of the potential clinical harm, or seriousness, of the reaction to the identified substance. |
| code CodeableConcept [0..1] | Code for an allergy or intolerance statement (either a positive or a negated/excluded statement). This may be a code for a substance or pharmaceutical product that is considered to be responsible for the adverse reaction risk (e.g., "Latex"), an allergy or intolerance condition (e.g., "Latex allergy"), or a negated/excluded code for a specific substance or class (e.g., "No latex allergy") or a general or categorical negated statement (e.g., "No known allergy", "No known drug allergies") |
| patient Reference | The patient who has the allergy or intolerance. |
| encounter Reference [0..1] | The encounter when the allergy or intolerance was asserted. |
| onset [x] [0..1] | Estimated or actual date, date-time, or age when allergy or intolerance was identified. |
| recordedDate dateTime [0..1] | The recordedDate represents when this particular AllergyIntolerance record was created in the system, which is often a system-generated date |
| recorder Reference [0..1] | Individual who recorded the record and takes responsibility for its content. |
| asserter Reference [0..1] | The source of the information about the allergy that is recorded. |
| lastOccurrence dateTime [0..1] | Represents the date and/or time of the last known occurrence of a reaction event. |
| note String [0..*] | Additional narrative about the propensity for the Adverse Reaction, not captured in other fields. |

#### Reaction (Class)

Details about each adverse reaction event linked to exposure to the identified substance.

| Attribute | Notes |
| --- | --- |
| substance CodeableConcept [0..1] | Identification of the specific substance (or pharmaceutical product) considered to be responsible for the Adverse Reaction event |
| manifestation CodeableConcept [1..*] | Clinical symptoms and/or signs that are observed or associated with the adverse reaction event. |
| description String [0..1] | Text description about the reaction as a whole, including details of the manifestation if required. |
| onset dateTime [0..1] | Record of the date and/or time of the onset of the Reaction. |
| severity code [0..1] | Clinical assessment of the severity of the reaction event as a whole, potentially considering multiple different manifestations. |
| exposureRoute CodeableConcept [0..1] | Identification of the route by which the subject was exposed to the substance. |
| note Annotation [0..*] | Additional text about the adverse reaction event not captured in other fields. |

### Site Preparation

Figure 21 - Site Preparation

Preparation of a body site prior to an invasive or surgical procedure or the application of electrodes.

#### SitePreparation

| Attribute | Notes |
| --- | --- |
| preWash boolean | Washing the skin / showering before the site preparation can reduce the bacterial population on the skin. |
| degreased boolean | If an electrode is to be applied an alcohol wipe may be used to degrease the surface |
| abraded boolean | If an electrode is to be applied light abrasion with pumice or an abrasive cream may be used to reduce electrical resistance |

#### AntisepticApplication (Class)

Antiseptic agents such as alcohol, chlorhexidine, triclosan and iodine contain agents that can rapidly kill both resident and transient microorganisms. Some agents are also able to suppress their regrowth for the duration of the surgical procedures.

https://www.onetogether.org.uk/downloads/Surgical%20Skin%20Preparation%20Quality%20Improvement%20Guide_AW.pdf

| Attribute | Notes |
| --- | --- |
| antiseptic CodeableConcept | The antiseptic used |
| duration Period | The duration of the application process |
| applicationSwab boolean | True if the antiseptic is applied with a swab |
| applicationSpray boolean | True if the antiseptic is sprayed onto the skin surface |
| allowedToDry boolean | True if the antiseptic solution is allowed to dry before proceeding. This can increase its effectiveness. |

#### Drape (Class)

Surgical drapes reduce the risk of bacteria entering the sterile field. They may be full or partial and may have a transparent panel inset.

| Attribute | Notes |
| --- | --- |
| full boolean | True if the drape is full |
| partial boolean | True of the drape is partial |
| fenestrated boolean | True if the drape incorporates a transparent panel |

### Positioning

Figure 22 - Position

#### Positioning Aid (Class)

Positioning aids range from simple pads to complex frames. They can be used to maintain a position and also to protect the patient by padding pressure points and reducing the risk of peripheral nerve damage.

Perioperative peripheral nerve injury (PPNI) causes significant patient morbidity and is a leading cause of anesthesia-related litigation.

Mechanisms of injury include compression, stretch ischemia, direct nerve trauma, and local anesthetic toxicity (https://academic.oup.com/bjaed/article/12/1/38/260058)

| Attribute | Notes |
| --- | --- |
| aidType CodeableConcept | The general class of aid e.g. wedge, frame, board |
| aidDescription CodeableConcept | The actual aid used |
| placement CodeableConcept | The body site where the aid is placed |
| roleMaintenance boolean | True if the aid is used to maintain the position of the body, or body part. This does not exclude other purposes/functions of the device |
| rolePatientProtection boolean | True if the aid is used to protect the patient. This does not exclude other purposes/functions of the device |
| rolePressureRelief boolean | True if the aid is used very specifically to reduce pressure at a pressure point. This does not exclude other purposes/functions of the device |
| startTime dateTime | The time the aid was placed |
| endTime dateTime | The time the aid was removed |

#### Body Position (Class)

The overall position of the patient's body. Some common positions are:

- Supine

- Prone

- Left lateral

- Right lateral

- Knee-Chest

- Trendelenberg

- Reverse trendelenberg

- Jacknife

- Fowlers

| Attribute | Notes |
| --- | --- |
| bodyPosition CodeableConcept | Some procedures require the whole body to be tilted or rotated e.g Trendelenburg. The degree of tilt can have implications for anesthetic management |

#### Arm Position (Class)

The positioning of the arms is important in anesthesia because the arms contain a number of preferred sites for IV access and are also conveniently out of  surgeons' way. The arm position is defined in terms of two components: the shoulder and the elbow.

| Attribute | Notes |
| --- | --- |
| description Arm Position |  |

#### Elbow (Class)

The position of the elbow does not require to be known with precision.

| Attribute | Notes |
| --- | --- |
| laterality CodeableConcept | The laterality of the elbow, left or right |
| elbowBent boolean | True if the elbow is bent |

#### Head and Neck (Class)

The head and neck may be specifically positioned for some procedures. The 'sniffing' position used during intubation is an example and positioning for cranial surgery is another.

| Attribute | Notes |
| --- | --- |
| neckNeutral boolean | The normal position of the neck in a natural supine position |
| neckFlexion boolean | Neck flexion is involved when the head bends towards the chest |
| neckExtension boolean | Neck extension is when the head bends backwards |
| sniffingPosition boolean | This position is often used for intubation as it aligns the axes of the oral cavity, pharynx and trachea. |
| headRotationLeft boolean | Head rotated around the vertical axis to the left |
| headRotationRight boolean | Head rotated around the vertical axis to the right |
| headRotation Quantity | The degrees of rotation, left or right. Straight ahead is 0. |

#### Leg Position (Class)

The position of the legs is not typically of relevance in anesthesia except insofar as it reflects a body position such as the lithotomy position. Both hips and knees can be flexed and the upper legs, abducted but this is not commonly recorded in the anesthesia record

#### PatientPositioning (Class)

The positioning of the patient and parts of the body is one of the acts preparatory to most procedures. Position may be changed throughout the encompassing episode to allow surgical access, to enable specific anesthetic procedures and to avoid problems associated with some positions being maintained for an excessive time.

| Attribute | Notes |
| --- | --- |
| startTime dateTime | The time at which the position is established |
| endTime dateTime | The time at which the position is changed or ceases |
| annotation Annotation | Free text annotation |

#### Shoulder (Class)

This defines the position of the upper arm relative to the body.

| Attribute | Notes |
| --- | --- |
| laterality CodeableConcept | The laterality of the arm, left or right. |
| tucked boolean | The arm may be stabilized by being tucked against the side using a sheet. This is referred to as being tucked |
| neutral boolean | The arm is in a natural position in relation to the side |
| abducted boolean | The arm is angled away from the side |
| abductedL>90degrees boolean | The angle of abduction is less than 90 degrees |
| abductionAngleDegrees Quantity | The abduction angle in degrees from 0 (by the side) to 180 (above the head). |

#### Table Tilt (Class)

For some procedures the operating table is tilted. Ana example is when the Trendelenburg position is used

| Attribute | Notes |
| --- | --- |
| degreesHeadUp Quantity | The number of degrees by which the head is tilted up from the horizontal |
| degreesHeadDown Quantity | The number of degrees by which the head is tilted down from the horizontal |
| degreesLeftTilt Quantity | The number of degrees by which the body is rotated to the left around the vertical axis |
| degreesRightTilt Quantity | The number of degrees by which the body is rotated to the right around the vertical axis |

### Aseptic Technique

#### AsepticTechnique (Class)

Aseptic technique refers to a collection of actions that mitigate the risk of a caregiver transmitting infection to a patient.

| Attribute | Notes |
| --- | --- |
| asepticTechnique boolean | True if standard aseptic technique is observed |
| ringsRemoved boolean | True if rings are removed |
| handWashing boolean | True if hands are washed |
| watchBraceletRemoved boolean | True if watches, bracelets etc. are removes |
| antisepticAgent CE [0..*] | True if an antiseptic agent is applied in addition to hand washing |
| mask boolean | True if a mask is worn |
| gown boolean | True if a gown is worn |
| surgicalCap boolean | True if a surgical cap is worn |
| gloves boolean | True if gloves are worn |
| surgicalScrubs boolean | True if surgical scrubs are worn |

### Kit

Figure 23 - Kit

#### Kit (Class)

A pre-assembled collection of items appropriate for the performance of a procedure

| Attribute | Notes |
| --- | --- |
| identifier Identifier [0..*] | Unique instance identifiers assigned to a device by manufacturers other organizations or owners. It is anticipated that this will be taken from a barcode on the kit |
| type CodeableConcept [0..1] | What kind of kit this is e.g. IV set, continuous nerve block set etc. |
| manufacturer string [0..1] | Name of manufacturer |
| name string | The name of the kit e.g. Braun Spinocan® Spinal Anesthesia Tray |
| definition Reference | The reference to the definition for the kit possibly held on an inventory system. This provides a means to identify the individual components of the kit from which the use of particular devices and/or medications might be inferred. |
| note Annotation [0..*] | A free text note |
| safety CodeableConcept [0..*] | Safety characteristics of kit. For example, whether or not the kit contains particular materials such as latex, BPA, DEHP, PVC and if the components are MRI-safe. |

### Device

The concept of a Device spans everything from a hypodermic needle to a multi-function patient monitor. It follows that a model appropriate for the latter is over-engineered for the former. This means that many if not most, cases in anesthesia relatively few attributes are required.

Figure 24 - Device

#### Device (Class)

A type of a manufactured item that is used in the provision of healthcare without being substantially changed through that activity. The device may be a medical or non-medical device (From:FHIR:Device).

| Attribute | Notes |
| --- | --- |
| identifier Identifier [0..*] | An instance identifier. In the Anesthesia DAM this would be used to identify a unique instance of a device, used during an anesthetic procedure. For example, a catheter placed in the dorsum of the left hand might need to be linked with a medication administration. The identifier could provide this link. |
| definition ref [0..1] | Type: Reference (DeviceDefinition) The reference to the definition for the device -the "catalog" definition of a device - whether that definition is authored by the regulatory entities, or it is a local definition that includes assembled device configurations. The DeviceDefinition allows defining hierarchical device configurations (devices as part of other devices). The Device resource is meant to refer to a physical instance of a device - hence having attributes like lot number, patient, location, which the DeviceDefinition resource does not. |
| type CodeableConcept [0..1] | Type: Codeable Concept (SNOMED CT) What kind of device this is, for example, needle, laryngoscope, tube, catheter, patient monitor. |
| status code [0..1] | - Status of the Device availability. - Active - The device is available for use. - inactive - The device is no longer available for use (e.g. lost, expired, damaged). |
| statusReason CodeableConcept [0..1] | Type: CodeableConcept Reason for the status of the Device availability. For example, an anesthetic machine might not be available/useable , as a result of a fault being discovered during the machine checklist. |
| distinctIdentifier char [0..1] | Type: string The distinct identification string as required by regulation for a human cell, tissue, or cellular and tissue-based product. |
| safety CodeableConcept [0..*] | Type string: Safety Characteristics of Device, provides additional information about the safety characteristics of a medical device. For example, devices containing latex, MRI-safe etc. Other examples would be laser resistant surgical drapes and ET tubes. This property is often implied in a pre-coordinated concept name in SNOMED CT - rather than by separate qualifier e.g. SCTID: 470636009 Laser-resistant surgical drape. |
| parent Reference(Device) [0..1] | Type: Reference (Device) The parent device. For example, a Macintosh laryngoscope blade would have a parent laryngoscope to which this reference would point. Similarly, an anesthetic vaporizer might have a reference to the anesthetic machine to which it was attached. This allows complex devices to be defined. |
| note Annotation [0..*] | Type: Annotation A free text annotation to record information not captured by the attributes already defined. |
| patient Reference(Patient) [0..1] | Type: reference In the context of an anesthetic episode, the patient identity is asserted in the patient demographics and typically applies to all artifacts where the identity of the patient is recorded. It does not normally require to be entered separately for each instance. |
| url uri [0..1] | Network address to contact device. This is used in the case of an electronic anesthetic record where data from patient-connected device may be acquired via a monitoring network. |

#### UDI (Class)

Unique Device Identifier

| Attribute | Notes |
| --- | --- |
| deviceIdentifier string [0..1] | Unique instance identifiers assigned to a device by manufacturers other organizations or owners. The barcode string from a barcode present on a device label or package may identify the instance, include names given to the device in local usage, or may identify the type of device. If the identifier identifies the type of device, Device.type element should be used. |
| jurisdiction uri [0..1] | Regional UDI authority |
| carrierHRF string [0..1] | UDI Human Readable Barcode String |
| carrierAIDV base64binary [0..1] | UDI Machine Readable Barcode String |
| issuer uri [0..1] | UDI Issuing Organization |
| entrytype code [0..1] | Barcode/rfid/manual + UDIEntryType |

#### DeviceName (Class)

This represents the type or provenance of the name for example:

- UDI Label name.

- User Friendly name

- Patient Reported name

- Manufacturer name.

- Model name

- Other

| Attribute | Notes |
| --- | --- |
| name string | The name of the device, typically the name assigned by the manufacturer |
| type | The type of deviceName. e.g. - UDILabelName - UserFriendlyName - PatientReportedName ManufactureDeviceName - ModelName For example, a model of anesthetic machine manufactured by Draeger is the Perseus 500 so the deviceName 'type' would be Model and the DeviceName 'name' might be Draeger Perseus 500 |

#### Specialization (Class)

The capabilities supported on a device, the standards to which the device conforms for a particular purpose and used for the communication.

| Attribute | Notes |
| --- | --- |
| systemType string | The standard that is used to operate and communicate |
| version | Type: string The standard that is used to operate and communicate. |

#### Version (Class)

The actual design of the device or software version running on the device. This is important in anesthesia where problems can be caused by obsolete drivers or record keeping software

| Attribute | Notes |
| --- | --- |
| type [0..1] | Type: CodeableConcept The type of the device version. |
| component [0..1] | A single component of the device version. |
| value | Type: string The version text |

#### Property (Class)

The actual configuration settings of a device as it actually operates, e.g., regulation status, time properties.

| Attribute | Notes |
| --- | --- |
| type | Type: CodeableConcept Code that specifies the property DeviceDefinitionPropertyCode (Extensible). |
| valueQuantity | Type: Quantity Property value as a quantity. |
| valueCode [0..*] | Type: CodeableConcept Property value as a code, e.g., NTP4 (synced to NTP) |

#### Material (Class)

The material of which the device, or part of the device is made. This has implications for the avoidance of allergic reactions and also for the physical properties of the device that may affect safety.

| Attribute | Notes |
| --- | --- |
| substance CodeableConcept | The substance |
| alternate CodeableConcept | Indicates an alternative material of the device |
| allergenicIndcator boolean | Whether the substance is a known or suspected allergen |

### Needle

Figure 25 - Needle

#### Needle (Class)

In anesthesia many of the attributes of a needle are implicit in the name. There are important differences between types of needle used for epidural, spinal and regional blocks. A particularly important characteristic is the configuration of the needle tip. Other features are calibration (depth markings which allow the depth of insertion to be gauged, and hence information such as the distance of the epidural space from the skin), echogenesis (which affects the ease with which the needle can be visualized using ultrasound), insulation (that is needed when peripheral nerve stimulation is used to assess a block), calibration (depth markings which allow the depth of insertion to be gauged, and hence information such as the distance of the epidural space from the skin), wings (which aid insertion and stability) and the connector type.

| Attribute | Notes |
| --- | --- |
| calibrated boolean | Calibrated needles have markings that allow the distance the needle has penetrated to be known |
| echogenic boolean | Ultrasound guidance is now commonly used when inserting regional block needles. However, the tip position of standard needles may not be easy to see. To improve visibility the needle surface can be modified and/or a special coating applied that improves the backscattering of ultrasound. A needle of this type is described as echogenic. |
| wings boolean | Wings are projections to either side of the needle at its proximal end. They can facilitate good control of the needle as it is inserted and can also be used to stabilize the needle when it is left in place and secured. |
| insulated boolean | A nerve stimulator can be used to generate a motor response the presence or absence of which can signify a satisfactory degree of nerve block. A needle insulated with Teflon or silicone focuses the stimulation current density on the needle tip so allowing precise localization of the stimulus. |
| intendedAgeRange enum | - None - Pediatric - Adult |

#### TipConfiguration (Class)

The tip configuration may have significant implications. For example, the incidence of post-dural puncture needle headache or the ease with which a procedure is accomplished. The tip characteristics are associated with the needle type. Many needle tips feature a bevel and here the angle of the bevel is important, for example, a shallow angle results in less trauma to the dura while some regional block needles have a specialized 45-degree bevel. Where relevant the specifics of the tip type are noted in the relevant vocabulary elements.

| Attribute | Notes |
| --- | --- |
| tipForm CodeableConcept | The form of the tip, for example, round, beveled, pencil point |
| sideHole boolean | A side hole or side port is an opening in the side of the needle usually, close to the tip. For example, a pencil point needle has a solid, sharp pointed tip, and a side hole that allows fluid to exit the needle into an adjacent structure. |
| angleOfBevel Quantity | The angle of bevel of the needle. Spinal needles often feature shallow angled bevels which may reduce dural trauma. Other needles, used in regional anesthesia, have a short (45-degree bevel) which may reduce the risk of nerve injury. |
| backEye boolean | A back eye is a feature of some epidural needles (Tuohy) that is used in a combined spinal/epdural technique. It is a hole in the side of the needle at the distal end where the tip has a slight curve. A spinal needle can use the epidural needle as a physical guide and introducer, passing through epidural needle and exiting via the back eye, when it is advanced into the subarachnoid space. |

### Catheter

Figure 26 - Catheter

#### Catheter (Class)

| Attribute | Notes |
| --- | --- |
| Connector Type CodeableConcept | The connected type e.g. - Luer lock - Luer slip - NR Fit - EN Fit |
| Wings boolean | Denotes whether the catheter has wings or not. |
| Hub boolean | Denotes whether the catheter has a hub or not. |
| intendedAgeRange enum | The intended age range of patients for which the catheter has been designed. - None - Pediatric - Adult |
| intendedGender CodeableConcept | The intended gender of patients, male/female for which the catheter has been designed. This attribute normally applies only to urinary catheters |

#### Epidural and Spinal Catheters (Class)

Spinal catheters/microcatheters are used in Continuous Spinal Anesthesia (CSA) and are generally much thinner than epidural catheters .

| Attribute | Notes |
| --- | --- |
| antiBacterialFilter boolean | An anti-bacterial filter may be fitted to the proximal end of the catheter to minimize the risk of infection |
| deflectable boolean | Some catheters have the ability to steer the tip using a pull wire that runs the length of the catheter. These are known as deflectable. The feature is particularly useful in the case of very fine catheters where the direction of the tip within the target structure is less easy to guarantee |
| endHole boolean | The catheter has a hole at its end |
| sideHole boolean | The catheter has one or more holes at its side |
| numberHoles int | The number of holes |

### Tube

Figure 27 - Tube

#### Tube (Class)

| Attribute | Notes |
| --- | --- |
| category CodeableConcept | - Epidural - Peripheral Intravenous - Central Venous - Peripheral Arterial |
| connectorSize Quantity | ET tubes will normally have either a 15 mm or 22 mm diameter |
| hub boolean |  |
| wings boolean |  |
| lateralization CodeableConcept | Some tubes, for example, double-lumen endobrochial tubes, are described as 'right' or 'left' sided according to which main bronchus they are designed to intubate |
| intendedSex CodeableConcept | - Male - Female |

#### Tracheal Tube (Class)

| Attribute | Notes |
| --- | --- |
| numCuffs CodeableConcept | The ET tube can be orientated so that as it exits the patient it points towards the head, feet or else in no particular direction. Particularly in the case of some preformed tubes the orientation will be linked to the preformed angle. Terms used to describe the orientation are: - Neutral - North-cephalad - South - caudad |
| orientation Quantity | ET tubes will normally have either a 15 mm or 22 mm diameter |
| intendedInsertionSite boolean | This is either Oral or Nasal |
| aboveCuffSuctionPort boolean | Tracheal tubes with a suction port above the cuff can assist in reducing the rate of ventilator-associated pneumonia. |
| belowCuffSuctionPort boolean | Allows suctioning of secretions distally |
| murphyEye CodeableConcept | A Murphy eye is an opening in the side of the tube that allows gases to pass in the event that the tip is occluded |
| laserResistant CodeableConcept | Airway tubes can be damaged by surgical lasers and in extreme cases may catch fire. Some tubes are specially treated, or foil wrapped to prevent this. |

### Mask

Figure 28 – Mask

#### Mask (Class)

| Attribute | Notes |
| --- | --- |
| category CodeableConcept | - Face - Nasal - Re-breather |
| connectorSize Quantity | Normally either 15 mm or 22 mm |
| transparent boolean |  |
| size CodeableConcept |  |

#### Face Mask (Class)

| Attribute | Notes |
| --- | --- |
| sealType CodeableConcept | The seal is the portion of the mask that is in contact with the patient's face. It establishes the low-pressure seal and creates a snug fit. Values for sealType are: - cuffed - pad - cushion |

Figure 29 - Tourniquet

#### Pressure (Class)

The tourniquet cuff (or cuffs) can be inflated or deflated. Any change from one state to the other has an associated time period and cuff pressure. It is thus possible to know for how long a cuff has been inflated and at what pressure. The pressure is relieved at intervals to avoid tissue damage. In the case of a dual-cuff tourniquet, cuffs are inflated and deflated alternately.

| Attribute | Notes |
| --- | --- |
| effective instant | The time at which an inflation or deflation is performed |
| inflation boolean | Indication that an inflation was done at this time |
| deflation boolean | Indication that a deflation was done at this time |
| initialPressure Quantity [0..1] | The pressure in the cuff preceding an inflation or deflation |
| newPressure Quantity [0..1] |  |
| leakPressure Quantity |  |

#### Alarm (Class)

On automatic systems it is possible to set an alarm that alerts the operator when the tourniquet has been inflated for a certain time. This assists in the prevention of tissue damage and tourniquet pain. As a general rule a tourniquet should not be left inflated for more than 2 hours and any time over 1 hour may be associated with problems.

| Attribute | Notes |
| --- | --- |
| pressure Quantity | A pressure level at which the alarm will sound |
| alarmSetTime dateTime | The time when the alarm is set |
| elapsedTime Period | The elapsed time after setting that the alarm should sound |

### Laryngoscope

A laryngoscope is a device used to visualize the larynx to facilitate endotracheal intubation. The basic structure comprises a handle, that often contains a power source and sometimes a light source (Green Standard) and a blade.

#### Laryngoscope (Class)

| Attribute | Notes |
| --- | --- |
| bladeDescription CodeableConcept | The name of the blade e.g. Miller, Macintosh |
| bladeProfile CodeableConcept | Whether the blade is curved or straight |
| bladeSize CodeableConcept |  |
| bulbInBlade boolean | Whether the blade contains a light within it |
| greenStandard boolean | An ISO standard for laryngoscopes and blades |
| videoTip boolean | Video laryngoscopes contain a small camera in the tip |
| steerableTip boolean | A tip that can be manipulated independently by the operator |
| leftHandBlade boolean |  |
| flexibleBlade boolean |  |

### Lumen

Figure 30 - Lumen

Lumen refers to the bore of a tube (as of a hollow needle or catheter). Devices can have one or more lumens.

The lumen of larger devices such as endotracheal tubes is usually expressed as a unit measurement of the internal diameter e.g. 6 mm.

#### Lumen (Class)

| Attribute | Notes |
| --- | --- |
| connectorType string | ISO 8039 covers small-bore connectors for liquids and gases in medical applications. The standard is specialized by suffixes as follows: - ISO 80369-2 : Connectors for breathing systems and driving gases applications - ISO 80369-3 : Connectors for enteral applications - ISO 80369-4 : Connectors for urethral and urinary applications - ISO 80369-5 : Connectors for limb-cuff applications - ISO 80369-6 : Connectors for neuraxial applications - ISO 80369-7 : Connectors for intravascular or hypodermic applications ISO 80369-6 was developed to avoid accidental cross connection of devices which might allow, for example, an IV drug to be administered intrathecally during a neuraxial block. Descriptively, connectors are often referred to by type viz. Luer Lok, Luer slip, NRFit and EN fit. |
| color CodeableConcept | The color can be used to identify a specific lume in a multi-lumen device for example an airway, or multi-lumen catheter |
| nonReturnValve CodeableConcept | Both catheters and tubes may feature a non-return valve for example, a urinary catheter |
| positionOfExit boolean | The position of the exit in relation to the lumen. Examples: - Distal - Medial - Proximal |
| depthMarkings boolean | Depth markings can be used to determine the position of the tip of a device in relation to the entry point, for example the length of an epidural catheter in the epidural space, or of an ET tube from the incisors. |
| reinforced boolean | Catheters and tubes may be reinforced with wire or foil which can reduce the risk of kinking or collapse and may be necessary to give resistance to damage by lasers. |
| radioOpaque boolean | A lumen may be made radio-opaque by the addition of a radio-opaque strip or stripe which assists in guidance and verification of position when using X-ray/flouroscopy |
| laterality CodeableConcept | This is the laterality of this lumen in a tube that has a designated laterality e.g. a double lumen endo-bronchial tube |

#### Lumen Metrics (Class)

Where metrics are measured in physical units the type of unit should be noted. Length and measurements of diameter are examples.

| Attribute | Notes |
| --- | --- |
| gaugeSystem CodeableConcept | The gauge system, for example: - French - Standard Wire Gauge |
| gaugeValue CodeableConcept | The value of the gauge within the gauge system, for example, 18G |
| gaugeValueNom string | A nominated gauge value |
| manufacturedLength Quantity | The length of the lumen as supplied by the manufacturer |
| cutLength Quantity | Sometimes a device will be cut to a desired length. An example is where a central venous line is cut to the length estimated to be needed to reach the target site. This attribute does not apply to needles. |
| internalDiameter Quantity | The internal diameter of the lumen |
| external Diameter Quantity | The external diameter of the lumen |

#### Cuff (Class)

Inflatable cuffs are used to provide a good seal between the catheter and the walls of a vessel.

| Attribute | Notes |
| --- | --- |
| cuffPosition CodeableConcept | The position of the cuff on the catheter. Example: - Distal - Medial - Proximal |
| highPressLowVol boolean | A narrow, low volume cuff, inflated to a high pressure can provide a better seal. It may, however, cause some tissue trauma. |
| lowPressHighVol boolean | A wider, low pressure cuff may cause less trauma to tissues but the leak pressure may be lower |
| volume quantity | The inflated volume of the cuff |

### Positioning Aid

Positioning aids are used to maintain a head/neck or body position or the position of a limb. They are also used to alleviate pressure in order to avoid nerve or other injury. Examples are:

Andrews Frame, Arm cradle, Armrest, Axillary roll, Foam headrest donut, Gel chest rolls, Gel head support, Hip vice, Horseshoe headrest, Jackson table (spinal), Padding, Peg board, Pillow, Prone foam head support, Sheet, Stirrups, Vac pack, Wilson frame.

### Device Settings

Figure 31 - Device Settings

#### DeviceSettings (Class

These are settings for the device

| Attribute | Notes |
| --- | --- |
| identifier identifier | This is a business identifier, not a resource identifier |
| source Reference (Device) | The identity of the device to which this instance of 'Device Metric' relates. |
| parent Reference (Device) | Describes the link to the Device that this DeviceMetric belongs to and that provides information about the location of this DeviceMetric in the containment structure of the parent Device. An example would be a Device that represents a Channel. This reference can be used by a client application to distinguish DeviceMetrics that have the same type, but should be interpreted based on their containment location. |
| operationalStatus code | Indicates current operational state of the device. For example: On, Off, Standby, etc. |
| period period |  |

#### Ultrasound (Class)

| Attribute | Notes |
| --- | --- |
| linearProbe boolean | Preferred for relativelt shallow structures and the most used in anesthesia |
| nonLinearProbe boolean |  |
| frequency Quantity | The frequency (Hz.) of the ultrasound signal |
| depth Quantity |  |

#### NerveStimulator (Class)

| Attribute | Notes |
| --- | --- |
| voltage Quantity |  |
| current Quantity |  |
| pulseWidth Quantity | The identity of the device to which this instance of 'Device Settings' relates. |
| pattern CodeableConcept | The pattern of stimulation: - single twitch (ST) - train-of four (TOF) - double burst stimulation (DBS) - tetanic stimulation |
| repetitionFreq Quantity |  |

#### Pneumatic Tourniquet (Class)

| Attribute | Notes |
| --- | --- |
| cuffNumber int |  |
| cuffPressure Quantity |  |

#### Anesthetic Vaporizer (Class)

| Attribute | Notes |
| --- | --- |
| agent CodeableConcept | The volatile anesthetic agent e.g. desflurane |
| concentration Quantity |  |

#### Ventilator (Class)

See Ventilator Mode for more information

| Attribute | Notes |
| --- | --- |
| mode CodeableConcept |  |
| modeSetting int |  |
| pattern CodeableConcept |  |
| triggerType CodeableConcept |  |
| triggerValue Quantity |  |
| assuredInflationType CodeableConcept |  |
| AITSetting Quantity |  |
| supportInflationType CodeableConcept |  |
| SITSetting Quantity |  |
| 2ndSupportInflationType CodeableConcept |  |
| 2ndSITSetting Quantity |  |

### Device Use

Procedural devices may be non-invasive (such as a pulse oximeter sensor) or invasive. An invasive device is any medical device that is introduced into the body, either through a break in the skin or an opening in the body. Examples are airway devices (such as endo-tracheal and endo-bronchial tubes, laryngeal mask airways and tracheostomy tubes), devices for vascular access (such as peripheral and central venous lines) and devices used in regional block procedures to administer local anesthetic agents, such as  needles and catheters.

Figure 32 - Device Use

### DeviceUse (Class)

Many procedural devices that are used during intra-procedural anesthesia are invasive in nature e.g. tubes, needles, catheters and so the details of the device use can be complex.

| Attribute | Notes |
| --- | --- |
| id Identifier |  |
| partOf Ref (Procedure) | A reference to a procedure of which this device use is a part |
| attachedTo Reference (Device) | Reference to another device to which this one is attached. For example, a pulse oximeter probe is connected to a pulse oximeter |
| deviceRole CodeableConcept | The role of the device e.g. respiratory support, medication administration, guidance, introducer, measurement, patient protection |
| attempt int | The number of this attempt to place the device. In the case of invasive devices, an attempt is defined as the complete withdrawal of the device and reinsertion at a different site. Adjustments of the device during insertion are considered part of the same attempt |
| technique CodeableConcept [0..*] | Any special technique employed; examples are: Seldinger - where a catheter is passed along a guide wire Needle through needle - used in combined spinal-epidural anesthesia when the spinal needle is inserted through an epidural needle |
| difficulty Annotation | A description of how difficult the placement was to accomplish |
| patientPosition CodeableConcept | The position of the patient and or body part during the device placement |
| note Annotation | A free text note |

#### DeviceRole (AssociationClass)

The role of the device in the DeviceUse instance.

| Attribute | Notes |
| --- | --- |
| substanceAdministration CodeableConcept | This is one of the most common roles for a device in anesthesia for example the administration of fluids and drugs |
| acquisitionOfSample CodeableConcept | One role of a device may gbe to allow sampling of a substance as when an arterial line is used to sample blood for blood gas analysis |
| physiologicalMeasurement CodeableConcept | A device may be used to measure waveforms, pressures etc. as in the case of a Pulmonary Arterial Catheter (PAC) |
| deviceGuidance CodeableConcept | In anesthesia a device is frequently used as a means to facilitate the insertion or placement of another device as when a guidewire or introducer is used to provide a physical guide for a catheter or tube. Other forms of guidance may be indirect, for example where a nerve stimulator is used to elicit a muscle twitch that indicates that, for example, a needle is close to its target. Finally, a device may be used to visualize or image anatomical structures during the placement of another device. For example, when a video laryngoscope is used for intubation. |

#### InsertionSite (Class)

| Attribute | Notes |
| --- | --- |
| bodySite CodeableConcept | The site where the device is inserted |

#### Approach (Class)

| Attribute | Notes |
| --- | --- |
| approach CodeableConcept | The description of the approach used for the invasive device placement e.g. median, para-median |
| inPlane boolean | When ultrasound is used to guide or verify the position of a device an 'in plane' approach is when the device is advanced along the path of the beam |
| outOfPlane boolean | When ultrasound is used to guide or verify the position of a device an 'out of plane' approach is when the device is advance across the path of the beam |

#### TargetSite (Class)

The intended target site, examples are:

- right atrium

- epidural space

- sub-arachnoid space

- brachial plexus

| Attribute | Notes |
| --- | --- |
| bodySite CodeableConcept |  |

#### DevicePosition (Class)

| Attribute | Notes |
| --- | --- |
| relativeToInsertionSite Quantity | The device position relative to the intended insertion site e.g. needle tip from skin, ET tube from incisors |
| relativeToTargetSite Quantity | The position of the device relative to the intended target site |
| lengthInSpace Quantity | A specialized measure used in epidural anesthesia - the length of the catheter in the epidural space |
| timeMeasured instant | The time when the position was measured |

#### Completion (Class)

A short note about how the device is secured (e.g. adhesive patch, tape),

labeled and any accessories fitted (e.g. bacterial filter)

| Attribute | Notes |
| --- | --- |
| antiBacterialFilter boolean | True if the device has an antibacterial filter attached |
| flushed boolean | True if the device, typically a cannula, was flushed after placement. |
| flushAgent CodeableConcept | What the device was flushed with e.g. saline |
| flushVolume Quantity | The volume of flushing agent used |
| securedWith CodeableConcept | Method by which the device is secured e.g. tape, adhesive patch |
| labelled Boolean | Invasive devices that will be left in place need to be labelled with, at minimum, the time and the date of placement |
| labeller Reference | Reference to the member of the care team who labelled the device |

### Medication

Figure 33 - Medication

#### Medication (Class)

This resource is primarily used for the identification and definition of a medication for the purposes of prescribing, dispensing, and administering a medication as well as for making statements about medication use.

| Attribute | Notes |
| --- | --- |
| identifier Identifier | Business identifier for this medication. This is a business identifier, not a resource identifier |
| code CodeableConcept [0..1] | A code that specifies this medication |
| status code [0..1] | Value set: - active - inactive - entered-in-error |
| manufacturer Reference [0..1] | Describes the details of the manufacturer of the medication product. This is not intended to represent the distributor of a medication product. |
| form CodeableConcept | Describes the form of the item. Powder; tablets; capsule. |
| amount Ratio [0..1] | Specific amount of the drug in the packaged product. For example, when specifying a product that has the same strength (For example, Insulin glargine 100 unit per mL solution for injection), this attribute provides additional clarification of the package amount (For example, 3 mL, 10mL, etc.). |
| specialFeature CodeableConcept | This describes a special characteristic of the formulation such as baricity (hypobaric, isobaric, hyperbaric) and molecular weight as in low-molecular weight heparin (LMWH) or un-fractionated heparin. |

#### Batch (Class)

Information that only applies to packages (not products).

| Attribute | Notes |
| --- | --- |
| lotNumber string [0..1] | The assigned lot number of a batch of the specified product. |
| expirationDate dateTime [0..1] | When this specific batch of product will expire. |

#### Ingredient (Class)

Identifies a particular constituent of interest in the product.

| Attribute | Notes |
| --- | --- |
| item CodeableConcept | The actual ingredient - either a substance (simple ingredient) or another medication of a medication. The ingredient may reference a substance (for example, amoxicillin) or another medication (for example in the case of a compounded product, Glaxal Base). |
| isActive boolean [0..1] | Indication of whether this ingredient affects the therapeutic action of the drug. True indicates that the ingredient affects the therapeutic action of the drug (i.e. active). False indicates that the ingredient does not affect the therapeutic action of the drug (i.e. inactive). |
| strength Ratio [0..1] | Specifies how many (or how much) of the items there are in this Medication. For example, 250 mg per tablet. This is expressed as a ratio where the numerator is 250mg and the denominator is 1 tablet. |

### Medication Administration

Figure 34 - Medication Administration

#### Medication Administration (Class)

Describes the event of a patient consuming or otherwise being administered a medication. This may be as simple as swallowing a tablet or it may be a long running infusion. Related resources tie this event to the authorizing prescription, and the specific encounter between patient and health care practitioner.

The ReasonResult class records both the clinical intent behind the administration and the result. It is not intended to record the detail of any adverse reaction or event as a result of the medication administration. This is recorded separately as an adverse event which can be linked to the administration

| Attribute | Notes |
| --- | --- |
| identifier Identifier [0..*] | This is a business identifier, not a resource identifier Identifiers associated with this Medication Administration that are defined by business processes and/or used to refer to it when a direct URL reference to the resource itself is not appropriate. They are business identifiers assigned to this resource by the performer or other systems and remain constant as the resource is updated and propagates from server to server. |
| instantiates uri [0..*] | A protocol, guideline, orderset, or other definition that was adhered to in whole or in part by this event. |
| partOf Reference [0..*] | A larger event of which this particular event is a component or step |
| status code | Will generally be set to show that the administration has been completed. For some long running administrations such as infusions, it is possible for an administration to be started but not completed or it may be paused while some other process is under way. - in-progress - not done - on-hold - completed - enter-in-error - stopped - unknown |
| statusReason CodeableConcept [0..*] | A code indicating why the administration was not performed. |
| category CodeableConcept [0..1] | Indicates where the medication is expected to be consumed or administered Term bindings: - inpatient - outpatient - community |
| subject Reference | The person group receiving the medication. |
| context Reference | The visit, admission, or other contact between patient and health care provider during which the medication administration was performed. In this DAM this will normally be the intra-procedural anesthetic episode. |
| performer Reference | Indicates who or what performed the medication administration.. Note that this could be a syringe pump or a drip. |
| note Annotation [0..1] | Extra information about the medication administration that is not conveyed by the other attributes. |
| eventHistory Reference [0..*] | A summary of the events of interest that have occurred, such as when the administration was verified. |

#### Effective (Class)

A specific date/time or interval of time during which the administration took place (or did not take place, when the 'notGiven' attribute is true). For many administrations, such as swallowing a tablet the use of dateTime is more appropriate.

| Attribute | Notes |
| --- | --- |
| startAdmin dateTime | Starting time with inclusive boundary |
| endAdmin dateTime | End time with inclusive boundary, if not ongoing |
| periodAdmin Period | A time period defined by a start and end date/time. A period specifies a range of times. The context of use will specify whether the entire range applies (e.g. "the patient was an inpatient of the hospital for this time range") or one value from the period applies (e.g. "give to the patient between 2 and 4 pm on 24-Jun 2013"). |

#### Dosage (Class)

Describes the medication dosage information details e.g. dose, rate, site, route, etc.

When a device is used to administer the dose, there are two possible roles the device may have. One is as a line, where the device has been pre-placed and can be linked to the dosage by a reference to the device. The other is as an immediate use device where the device has not been pre-placed and will not remain in the patient. For example, a hypodermic needle used in a  simple subcutaneous injection is an immediate use device.

| Attribute | Notes |
| --- | --- |
| dose Quantity | The amount of the medication given at one administration event. Use this value when the administration is essentially an instantaneous event such as a swallowing a tablet or giving an injection. |
| text string | Free text dosage can be used for cases where the dosage administered is too complex to code. When coded dosage is present, the free text dosage may still be present for display to humans. |
| rate ratio | A ratio of two Quantity values - a numerator and a denominator + Rule: Numerator and denominator SHALL both be present, or both are absent. If both are absent, there SHALL be some extension present |
| site CodeableConcept | A coded specification of the anatomic site where the medication first entered the body. For example, "left arm". |
| route CodeableConcept | A code specifying the route or physiological path of administration of a therapeutic agent into or onto the patient. For example, topical, intravenous, etc. |
| line Reference (Device) [0..1] | A specific physical device by which the medication is administered as opposed to the generic route (IV, IM, Oral etc.). The device is identified by reference to its id. This allows, for example, the individual lumen of a multi-lumen catheter to be specifically identified as the means by which a medication is given. It references a device that has already been placed. |
| method CodeableConcept | A coded value indicating the method by which the medication is intended to be or was introduced into or on the body. This attribute will most often NOT be populated. It is most commonly used for injections. For example: Slow Push, Deep IV, LA infiltration. Another example is barbotage, the technique of repeated injection and aspiration of fluid as in the injection of a spinal anesthetic |

#### ReasonResult (Class)

This captures the 'clinical intent' behind the medication administration i.e. why the medication was given and the effect of the administration.

| Attribute | Notes |
| --- | --- |
| reasonCode CodeableConcept | A code indicating why the medication was given. |
| reasonReference Reference | Reference to a condition or observation that supports why the medication was administered. |
| resultCode CodeableConcept | The result / effect of the medication administration e.g. increase in blood pressure, satisfactory analgesia, satisfactory reversal of neuromuscular blockade. This attribute is not intended to record the detail of any adverse reaction to the medication administration which can be captured independently and in more detail as an 'Adverse Event'. |
| note Annotation | A free-text note |

#### Medication (Class)

This resource is primarily used for the identification and definition of a medication for the purposes of prescribing, dispensing, and administering a medication as well as for making statements about medication use.

| Attribute | Notes |
| --- | --- |
| identifier Identifier | Business identifier for this medication. This is a business identifier, not a resource identifier |
| code CodeableConcept [0..1] | A code that specifies this medication |
| status code [0..1] | Value set: active inactive entered-in-error |
| manufacturer Reference [0..1] | Describes the details of the manufacturer of the medication product. This is not intended to represent the distributor of a medication product. |
| form CodeableConcept | Describes the form of the item. Powder; tablets; capsule. |
| amount ratio [0..1] | Specific amount of the drug in the packaged product. For example, when specifying a product that has the same strength (For example, Insulin glargine 100 unit per mL solution for injection), this attribute provides additional clarification of the package amount (For example, 3 mL, 10mL, etc.). |

#### Batch (Class)

Information that only applies to packages (not products).

| Attribute | Notes |
| --- | --- |
| lotNumber string [0..1] | The assigned lot number of a batch of the specified product. |
| expirationDate dateTime [0..1] | When this specific batch of product will expire. |

#### Ingredient (Class)

Identifies a particular constituent of interest in the product.

| Attribute | Notes |
| --- | --- |
| item CodeableConcept | The actual ingredient - either a substance (simple ingredient) or another medication of a medication. The ingredient may reference a substance (for example, amoxicillin) or another medication (for example in the case of a compounded product, Glaxal Base). |
| isActive boolean [0..1] | Indication of whether this ingredient affects the therapeutic action of the drug. True indicates that the ingredient affects the therapeutic action of the drug (i.e. active). False indicates that the ingredient does not affect the therapeutic action of the drug (i.e. inactive). |
| strength Ratio [0..1] | Specifies how many (or how much) of the items there are in this Medication. For example, 250 mg per tablet. This is expressed as a ratio where the numerator is 250mg and the denominator is 1 tablet. |

# Vocabulary Examples

The following examples are taken from the IOTA anesthesia term set which is almost completely aligned with SNOMED CT (International) terms. In rare instances there is a term in the IOTA set that is not yet in SNOMED CT. These are indicated by the prefix ‘I –‘, below.

### Administrative procedure

SCTID -14734007 administrative procedure

| SCTID / IOTA ID | Description |
| --- | --- |
| 107724000 | patient transfer |
| 118662001 | medicolegal procedure |
| 129465004 | medical record |
| 182771004 | informed consent for procedure |
| 223458004 | informing |
| 223474009 | liaising with |
| 23745001 | documentation procedure |
| 304561000 | informing health care professional |
| 304562007 | informing doctor |
| 305056002 | admission procedure |
| 305335007 | admission to establishment |
| 305349003 | admission to department |
| 305351004 | admission to intensive care unit |
| 306561002 | discharge from establishment |
| 306562009 | discharge from service |
| 306564005 | discharge from anesthetic service |
| 306568008 | discharge from intensive care service |
| 306685000 | discharge to establishment |
| 308283009 | discharge from hospital |
| 308291000 | administrative transfer procedure |
| 308292007 | transfer of care |
| 311791003 | information gathering |
| 32485007 | hospital admission |
| 370786008 | confirming patient identity before operative/invasive procedure |
| 372039005 | patient identification |
| 372040007 | confirming patient identity prior to procedure |
| 37729005 | patient transfer in-hospital |
| 385775004 | informing nurse |
| 386317007 | health care information exchange |
| 397762001 | coding schema |
| 397769005 | unexpected admission to high dependency unit |
| 397794006 | anesthesia practitioner identified |
| 397821002 | patient transfer to intensive care unit |
| 397842000 | present during procedure |
| 397855004 | relieves |
| 397945004 | unexpected admission to intensive care unit |
| 397950005 | practitioner presence |
| 398007000 | regulatory documentation |
| 398035001 | joins case |
| 398047007 | immediately available throughout |
| 398068008 | patient transfer to postoperative anesthesia care unit |
| 398097006 | relief notes |
| 398108008 | present throughout the entire procedure |
| 398144003 | attending availability |
| 398154004 | QITTF (Quality Improvement Template Task Force) |
| 398159009 | all critical points |
| 398162007 | admission to high dependency unit |
| 398249003 | CMS supervisory note |
| 398255008 | AACD |
| 398315008 | exits case |
| 405614004 | unexpected hospital admission |
| 405624007 | administrative documentation |
| 405672008 | direct questioning |
| 405684005 | case initiator |
| 405685006 | case reliever |
| 406144005 | patient transfer to cardiac intensive care unit |
| 416115001 | confirming patient identity by direct questioning |
| 416287007 | confirming patient identity by guardian interview |
| 417336003 | confirming patient identity by wristband |
| 423289001 | informing surgeon |
| 427675001 | admission from establishment |
| 429202003 | transfer of care to hospital |
| 449214001 | transfer to high dependency unit |
| 50659003 | medical audit procedure |
| 55149002 | procurement of patient informed consent investigational study |
| 57537006 | utilization review |
| 58000006 | patient discharge |
| 86078004 | quality of care procedure |
| I-1033398612858 | milestone presence |
| I-1033401446903 | responsible |

### Patient Transfer

patient transfer in-hospital

| Attribute | Notes |  |
| --- | --- | --- |
| 397821002 | patient transfer to intensive care unit |  |
| 398068008 | patient transfer to postoperative anesthesia care unit |  |
| 406144005 | patient transfer to cardiac intensive care unit |  |
| 449214001 | transfer to high dependency unit |  |
| 308292007 |  | transfer of care |
| 429202003 |  | transfer of care to hospital |
| 86078004 |  | quality of care procedure |
| 50659003 |  | medical audit procedure |
| 57537006 |  | utilization review |
| 305056002 |  | admission procedure |
| 305335007 |  | admission to establishment |
| 32485007 |  | hospital admission |
| 405614004 |  | unexpected hospital admission |

### Airway device

SCTID – 43725001 airway device

| 398178009 | lighted stylette |
| --- | --- |
| 26412008 | endotracheal tube |
| 309801005 | armored endotracheal tube |
| 309803008 | Oxford endotracheal tube |
| 420673005 | Magill type endotracheal tube |
| 420344001 | Magill type oral endotracheal tube |
| 420855005 | Magill type nasal endotracheal tube |
| 430553000 | endotracheal tube with bronchial blocker |
| 442310009 | laser resistant endotracheal tube |
| 449298004 | endotracheal tube with directional tip |
| 420977008 | preformed endotracheal tube |
| 309802003 | RAE endotracheal tube |
| 309805001 | nasal RAE endotracheal tube |
| 309804002 | oral RAE endotracheal tube |
| 61330002 | nasopharyngeal airway device |
| 32667006 | oral airway |
| 360003007 | Guedel airway |
| 429375002 | esophageal tracheal double lumen supraglottic airway device |
| 398105006 | endobronchial tube |
| 440625007 | double lumen tracheobronchial tube |
| 360143004 | endobronchial blocker |
| 297120004 | anesthetic face mask |
| 442398003 | nasal mask |
| 713535007 | supraglottic airway |
| 257268009 | laryngeal mask |
| 398251004 | intubating laryngeal mask airway |
| 714741006 | intubating laryngeal mask with visualization screen |
| 713537004 | reinforced laryngeal mask airway |
| 714740007 | laryngeal mask airway with posterior cuff and drainage lumen |
| 713536008 | laryngeal mask with preformed cuff |
| 257307004 | esophageal obturator airway |
| 463505005 | Esophageal-obturating airway reusable |
| 463757007 | Esophageal-obturating airway single-use |
| 448348008 | airway exchange catheter |
| 449301000 | endotracheal ventilation catheter |
| 714742004 | bronchoscope intubation catheter with ventilation connector |
| 26128008 | bougie |
| 449286009 | gum elastic bougie |
| 449326009 | single use intubation bougie |
| 33686008 | stylet |
| 468949001 | endotracheal tube stylet reusable |
| 468507004 | endotracheal tube stylet single-use |

### Intravenous anesthetic agent

SCTID – 373504006 intravenous anesthetic

| 373476007 | midazolam |
| --- | --- |
| 387423006 | propofol |
| 387218008 | etomidate |
| 373488009 | methohexital |
| 412605006 | thiopentone |
| 387448009 | thiopental sodium |
| 373464007 | ketamine |

### Local anesthetic nerve block in lower limb (enumeration)

SCTID 31161008 local anesthetic nerve block in lower limb

| 52516005 | sciatic nerve block |
| --- | --- |
| 231166003 | sciatic nerve block; anterior approach |
| 231168002 | sciatic nerve block; lithotomy approach |
| 231167007 | sciatic nerve block; posterior approach |
| 416743002 | sciatic nerve block; lateral approach |
| 231169005 | femoral nerve block |
| 231163006 | hip block |
| 231164000 | hip block by intracapsular approach |
| 231165004 | hip block by extracapsular approach |
| 231172003 | three in one block |
| 231170006 | lateral femoral cutaneous nerve block |
| 231171005 | obturator nerve block |
| 231173008 | common peroneal nerve block |
| 231175001 | ankle block |
| 231176000 | deep peroneal nerve block |
| 231177009 | superficial peroneal nerve block |
| 231174002 | saphenous nerve block |
| 231179007 | sural nerve block |
| 231178004 | posterior tibial nerve block |
| 231186004 | digital nerve block in foot |
| 231180005 | ring block in foot |
| 231181009 | ring block of great toe |
| 231182002 | ring block of second toe |
| 231183007 | ring block of third toe |
| 231184001 | ring block of fourth toe |
| 231185000 | ring block of fifth toe |
| 231187008 | common digital nerve block in the foot |