# Representing nested relationships in RF2 Format

a preliminary set of ideas that could be developed further into a  full proposal for using RF2 format for distributing “nested” definitions

Observables example

- Each row in the relationships table still will have a relationshipID (SCTID format)

- Each rel row represents a concept axiom:

  - if the “type” is an “is-a”, then it means: “source is-a target”

  - if the “type” is an object property, then it means: “source some type target”

- In the usual non-nested case, the source is a conceptID. This remains the same.

- Option 1: without anonymous concept IDs

- If the source is a RelationshipID, then the row modifies the target of the row identified by the RelationshipID.

- Option 2: with anonymous concept IDs

  - The source is always a concept ID, and some concept IDs are introduced in the relationships table (and possibly not included anywhere else)

- observableX === observableEntity and specifiedBy some

  - (observationProcedure and observes some

    - (feature of entity and (R1 some V1) and (R2 some V2) and (R3 some V3) ) )

Without anonymous concept IDs:

| relationshipID | source | type | target |
| --- | --- | --- | --- |
| relationshipID-a | observableX | isa | observableEntity |
| relationshipID-b | observableX | specifiedBy | observationProcedure |
| relationshipID-c | relationshipID-b | observes | featureOfEntity |
| relationshipID-d | relationshipID-c | R1 | V1 |
| relationshipID-e | relationshipID-c | R2 | V2 |
| relationshipID-f | relationshipID-c | R3 | V3 |

With anonymous concept IDs

| relationshipID | source | type | target |
| --- | --- | --- | --- |
| 1 | observableX | isa | observableEntity |
| 2 | observableX | specifiedBy | anonC-1 |
| 3 | anonC-1 | isa | observationProcedure |
| 4 | anonC-1 | observes | anonC-2 |
| 5 | anonC-2 | isa | featureOfEntity |
| 6 | anonC-2 | R1 | V1 |
| 7 | anonC-2 | R2 | V2 |
| 8 | anonC-2 | R3 | V3 |

To give a more specific example, consider LOINC 20505-4

Bilirubin:MCnc:Pt:Urine:Qn:Test strip

'observable entity'

and ('Specified By' some

('observation procedure'

and (Technique some 'Test strip')

and (Scale some Qn)

and ('Direct Site' some ‘Urine specimen’)

and (Observes some

('Feature of entity'

and ('Property Type' some MCnc)

and ('Inheres In' some Urine)

and (Towards some Bilirubin)))))

Using relationship IDs

| relationshipID | source | type | target |
| --- | --- | --- | --- |
| relationshipID-a | LOINC 20505-4 | isa | ’observable entity’ |
| relationshipID-b | LOINC 20505-4 | specifiedBy | ‘observation procedure’ |
| relationshipID-c | relationshipID-b | Technique | Test strip |
| relationshipID-d | relationshipID-b | Scale | Qn |
| relationshipID-e | relationshipID-b | Direct Site | Urine specimen |
| relationshipID-f | relationshipID-b | Observes | Feature of entity |
| relationshipID-g | relationshipID-f | Property Type | MCnc |
| relationshipID-h | relationshipID-f | Inheres In | Urine |
| relationshipID-i | relationshipID-f | Towards | Bilirubin |

Using anonymous concept IDs

| relationshipID | source | type | target |
| --- | --- | --- | --- |
| 1 | LOINC 20505-4 | isa | ’observable entity’ |
| 2 | LOINC 20505-4 | specifiedBy | anonC-1 |
| 3 | anonC-1 | isa | ‘observation procedure’ |
| 4 | anonC-1 | Technique | Test strip |
| 5 | anonC-1 | Scale | Qn |
| 6 | anonC-1 | Direct Site | Urine specimen |
| 7 | anonC-1 | Observes | anonC-2 |
| 8 | anonC-2 | isa | Feature of entity |
| 9 | anonC-2 | Property Type | MCnc |
| 10 | anonC-2 | Inheres In | Urine |
| 11 | anonC-2 | Towards | Bilirubin |

Issues:

current RF2 allows only concept SCTIDs in the source field.

not clear whether a mixture of LOINC IDs and concept SCTIDs (and perhaps Relationship SCTIDs) would cause problems, and if so, how to revise this.